# DOGZILLA S2

Complete Developer Documentation

Yahboom DOGZILLA S2 · Raspberry Pi · ROS 2 Humble

Generated 2026-08-28
Repository: /home/pi/DOGZILLA
Embedded controller firmware source is not included; verified public interfaces and explicit unknowns are documented.

# Contents

- DOGZILLA S2 documentation

  - Start here

  - Specialized documents

  - Architecture at a glance

  - Source of truth order

  - Documentation maintenance rules

  - Rebuild and Git are separate

  - Rebuild the Word manual

- DOGZILLA S2 developer handoff

  - Read these documents in order

  - System summary

  - Non-negotiable invariants

  - Repository layout

  - Runtime modes

  - Source versus deployed code

  - Development and test workflow

  - Configuration sources of truth

  - Mapping and localization model

  - Navigation model

  - Web task model

  - Persistent and generated data

  - Logging and diagnosis

  - Safe change ownership

  - Current limitations

  - Handoff checklist

- DOGZILLA S2 software framework

  - System boundary

  - Hardware ownership

  - Coordinate frames

  - ROS components

  - Persistence

  - Operational status

  - Source map

- DOGZILLA S2 pipelines

  - 1. Mapping pipeline

  - 2. Localization pipeline

  - 3. Navigation pipeline

  - 4. Web mission pipeline

  - 5. Patrol pipeline

  - 6. Keepout pipeline

  - 7. Camera and RTAB-Map shadow pipeline

  - 8. Build and deployment pipeline

  - Failure boundaries

- DOGZILLA controller firmware and serial interface

  - What “firmware” means on this robot

  - Versions present on this Pi

  - Physical connection and ownership

  - Serial frame format

  - Public register map

  - Host-library scaling and limits

  - Public Python API by function

  - Preset action groups

  - IMU packet behavior

  - Battery and low-power behavior

  - Rest, stand and servo release

  - Original Yahboom application

  - Extending controller support safely

  - Unknowns that must stay labelled unknown

- DOGZILLA ROS 2 interface reference

  - Frames

  - Core topics

  - Actions

  - Services

  - Nodes by launch

  - Compose modes and devices

  - Web API summary

  - QoS and freshness expectations

- DOGZILLA S2 operations runbook

  - Physical preflight

  - Setup and health checks

  - Build and deployment model

  - Mapping a room

  - View an existing map without ROS

  - Localization without autonomous navigation

  - Mission Mode

  - Low-level navigation development

  - Camera and vision

  - IMU calibration and validation

  - Stand, rest and LiDAR power

  - Controller-only drive

  - Logs and diagnostics

  - Failure recovery

  - Safe shutdown

  - Backup checklist

- DOGZILLA computer vision

  - Implemented lessons

  - Architecture

  - Operator commands

  - Safety boundary and remaining lessons

- URDF and monocular RTAB-Map framework

  - Current status

  - Files

  - Camera contract

  - Measurements still required

  - Remaining activation gates

  - Validation status

- DOGZILLA S2 development roadmap

  - Safety rules for every milestone

  - P0: establish a dependable baseline

  - P1: calibrate the physical model

  - P2: mature autonomous missions

  - P3: experiments isolated from control

  - Firmware work boundary

  - Work explicitly not recommended

  - Suggested next three development cycles

Source chapter: docs/README.md

# DOGZILLA S2 documentation

This directory is the maintained technical handoff for the Yahboom DOGZILLA S2 on this Raspberry Pi. It documents the software that is present and the behavior that can be verified from source, public serial traffic and runtime interfaces.

The complete editable Word export is DOGZILLA_S2_COMPLETE_DEVELOPER_DOCUMENTATION.docx. It contains every maintained Markdown chapter in this directory plus the embedded four-layer pipeline diagram.

The embedded motor-controller firmware source is not available. These files do not pretend to reconstruct its private gait, power-button, low-battery or servo control internals. That boundary is documented explicitly so the next developer does not mistake a Python API for the firmware itself.

## Start here

| Document | Audience and purpose |
| --- | --- |
| DEVELOPER_HANDOFF.md | First read for a new maintainer: responsibilities, invariants, layout, runtime modes, testing and known limitations |
| OPERATIONS_RUNBOOK.md | Exact Pi-host commands for mapping, localization, missions, patrol, vision, calibration, diagnostics and safe shutdown |
| FIRMWARE_AND_SERIAL.md | Controller boundary, installed Yahboom library, serial framing/registers, scaling, actions, IMU, battery, stand/rest safety and unknown firmware internals |
| FRAMEWORK.md | Current hardware, container, ROS, web, vision and persistence architecture |
| PIPELINES.md | End-to-end data/control flows for mapping, localization, Nav2, web tasks, patrol, keepouts, vision and deployment |
| ROS_INTERFACES.md | Frames, topics, actions, services, nodes, Compose device ownership, web API and freshness rules |
| DEVELOPMENT_ROADMAP.md | Prioritized safety/reliability/calibration work with acceptance gates and explicit non-goals |

## Specialized documents

| Document | Scope |
| --- | --- |
| COMPUTER_VISION.md | Camera modes, detector models, patrol hazards, alerts and guarded Yahboom lesson integration |
| URDF_RTABMAP_MONO.md | Provisional URDF and isolated monocular RTAB-Map shadow experiment |
| DOGZILLA_PIPELINE_SLIDE.svg | Editable four-layer presentation diagram |
| DOGZILLA_PIPELINE_SLIDE.png | Rendered presentation image |

Related source-area guides:

- ROS package README

- deployment README

- web mission README

- calibration README

- model README

- navigation stability candidate

## Architecture at a glance

Hardware layer

MS200 LiDAR | DOGZILLA controller/IMU/battery/joints | mono camera

|

v

Perception layer

LaserScan | corrected IMU | image processing and patrol detections

|

v

SLAM/localization layer

Cartographer -> map, odom and robot pose

|

v

Navigation/application layer

web tasks -> Nav2 -> command filters -> safe_base -> controller

Cartographer is the SLAM implementation used by this project; “SLAM” is the general mapping/localization problem. Nav2 consumes the map and pose but does not create them. The camera perception pipeline observes objects and people; it is not the operational localization or collision authority.

## Source of truth order

When documentation and behavior disagree, investigate in this order:

- physical safety and observed hardware state;

- the currently running Docker image and complete session logs;

- current source plus configuration and tests;

- these maintained documents;

- historical Yahboom examples and notebooks.

The original app_dogzilla/ and Samples/ files are useful protocol and lesson references, but they bypass the ROS single-owner safety design and are not the operational runtime.

## Documentation maintenance rules

Update documentation in the same change when any of these change:

- device ownership, serial protocol or controller limits;

- frames, topics, actions, services or QoS/freshness gates;

- Docker services, volumes, image dependencies or operator commands;

- map/calibration/database formats;

- task states, browser/API behavior or authentication;

- model classes, patrol readiness or alert retention;

- safety behavior, physical acceptance status or known limitations.

Label facts as one of: verified from source, observed on this robot, prepared but inactive, experimental, or unknown. Never convert a guess into firmware documentation because it produced one successful motion.

## Rebuild and Git are separate

Editing source does not modify an already running container. dogzilla build creates a local Docker image, and restarting a mode deploys that image. Neither operation commits or pushes Git. Conversely, a Git commit backs up source but does not include Docker layers, task databases, calibration not committed by policy, or all robot-specific runtime data.

## Rebuild the Word manual

After changing a chapter, rebuild the combined document from the repository:

python3 docs/build_word_manual.py

python3 runs the installed interpreter and docs/build_word_manual.py reads the maintained chapter order, renders the Markdown, embeds the pipeline image, uses headless LibreOffice to create the DOCX, and validates the resulting Word package. It does not start Docker, ROS or robot hardware.

Source chapter: docs/DEVELOPER_HANDOFF.md

# DOGZILLA S2 developer handoff

This is the starting point for the next developer. It describes what is operational, where each responsibility lives, how changes reach the robot and which boundaries must not be crossed.

## Read these documents in order

- This handoff.

- FRAMEWORK.md for component boundaries and current status.

- PIPELINES.md for end-to-end data and command flows.

- FIRMWARE_AND_SERIAL.md before touching movement, posture, battery, actions, IMU or servos.

- ROS_INTERFACES.md before adding a node or topic.

- OPERATIONS_RUNBOOK.md before deploying or running a physical test.

- DEVELOPMENT_ROADMAP.md before selecting the next feature.

Specialized references:

- COMPUTER_VISION.md

- URDF_RTABMAP_MONO.md

- calibration/README.md

- web/README.md

- deploy/README.md

## System summary

The robot is a Yahboom DOGZILLA S2 controlled by a Raspberry Pi. This repository adds a reproducible ROS 2 Humble stack around the vendor controller:

MS200 LiDAR + controller telemetry + mono camera

|

v

ROS perception and Cartographer

|

v

map, localization and scan odometry

|

v

web missions -> Nav2 -> command filters -> safe_base -> controller

The operational navigation system is 2-D LiDAR based. Cartographer provides mapping and scan-matched localization. Nav2 provides path planning, live costmaps and path following. The camera provides observation and patrol alerts; it is not the operational localization authority. Monocular RTAB-Map remains a separate experimental shadow pipeline.

DOGZILLA has no wheel encoders and no contact sensor. /odom is calculated from Cartographer's scan-matched TF. Stall detection is therefore an inference from commanded motion, LiDAR and scan odometry, not proof that the robot hit an object.

## Non-negotiable invariants

- One controller serial owner. Only one process may open /dev/ttyAMA0. Extend safe_base; never add a competing controller node.

- One LiDAR owner. Mapping or navigation owns /dev/ttyAMA1.

- One camera owner. A camera service or the Yahboom app may own /dev/video0, never both.

- Final movement passes through safe_base. Browser, camera, mission and diagnostic code must not open serial or command motors directly.

- Stop is layered. Nav2 cancellation, zero velocity, Twist Mux, the steering guard, safe_base clamps/watchdog and controller stop each cover a different failure.

- Localization precedes autonomy. A task cannot dispatch until the live scan agrees with the selected map and pose.

- LiDAR geometry remains fixed. Body height/look controls are disabled during mapping and localization.

- Low battery wins. Do not override the controller's 25% protection.

- No guessed rest trajectory. rest remains disabled until a real, independently reviewed firmware trajectory and torque sequence are proven.

- No automatic Git or registry push. Build and deploy are local actions; commit/push require a separate explicit operator decision.

## Repository layout

| Path | Responsibility |
| --- | --- |
| README.md | Project entry point and common workflow |
| docs/ | Architecture, interfaces, operations and specialized design documents |
| deploy/dogzilla-map | Main host operator command; use this instead of raw Compose for normal work |
| deploy/dogzilla-mission | Starts/stops navigation, mission perception and web as one managed session |
| deploy/dogzilla-web | Internal web-container helper |
| deploy/compose.yaml | Container boundaries, devices, volumes, commands and health checks |
| deploy/Dockerfile | Reproducible ARM64 runtime image |
| deploy/ros-packages.lock | Exact apt packages installed in the image |
| deploy/patches/ | Reviewed upstream compatibility/safety patches |
| ros2/dogzilla_slam/launch/ | Combined ROS launch descriptions |
| ros2/dogzilla_slam/config/ | Cartographer, Nav2, camera, Twist Mux and RTAB configuration |
| ros2/dogzilla_slam/dogzilla_slam/ | ROS nodes and pure policy/state modules |
| ros2/dogzilla_slam/behavior_trees/ | Fail-closed Nav2 behavior trees |
| ros2/dogzilla_slam/urdf/ | Provisional visualization model |
| ros2/dogzilla_slam/test/ | Package unit and deployment-contract tests |
| deploy/test/ | Host mission coordinator tests |
| app_dogzilla/ | Original Yahboom mobile app, joystick, camera and OLED code |
| Samples/ | Original Yahboom notebooks and examples; not the safe runtime |
| maps/ | Persistent map bundles |
| calibration/ | Robot-specific IMU and camera measurements |
| models/ | Deployed detection models and labels |
| training/ | Offline model export/training helpers |
| profiles/ | Controller captures; captures are not replay approval |
| web/data/ | Runtime SQLite task database and bounded alert images |
| logs/ | Timestamped runtime sessions and diagnostic reports |
| plans/navigation-stability/ | Prepared, reversible navigation candidate; inactive unless explicitly applied |

## Runtime modes

| Operator mode | Containers | Moves robot? | Purpose |
| --- | --- | --- | --- |
| start | mapping | Teleop only | Build a Cartographer map |
| drive | drive | Teleop | Controller-only testing; LiDAR off |
| localize | navigation without Nav2 | No autonomous movement | Check pose against a saved map |
| navigate | navigation with Nav2 | Goal/teleop capable | Low-level navigation development |
| mission | navigation + perception + web | Web tasks | Normal delivery, route and patrol workflow |
| vision | vision + web | No | Detection-only camera lessons/lab |
| vision-control | vision_control | Yes, explicitly armed | Narrow allowlisted Yahboom camera behaviors |
| shadow | mapping + visual_shadow | Teleop only | Experimental Cartographer plus RTAB recording |

Only the operator wrapper should combine these modes. Raw launch files do not enforce all host service, device, log and cleanup checks.

## Source versus deployed code

Editing files under ros2/dogzilla_slam/ does not change a running container. The deployment path is:

edit source -> run tests -> build image -> stop old mode -> start mode -> verify

The image copies ros2/dogzilla_slam into /root/yahboomcar_ws/src/dogzilla_slam and runs colcon build. There is no source bind mount for the package. A stale image can therefore look healthy while still running older code.

Before work, always inspect:

cd /home/pi/DOGZILLA

git status --short

cd changes the terminal's working directory to the repository. git status reports tracked and untracked changes. --short uses the compact two-column format. Existing changes may belong to another developer; do not reset or overwrite them.

## Development and test workflow

The bare Pi host does not contain the complete pytest and ROS Python test environment. After a local image exists, run the full source suite in a disposable, network-disabled container with the repository read-only and no robot devices:

sudo docker run --rm --network none \

--volume /home/pi/DOGZILLA:/workspace:ro \

--workdir /workspace dogzilla-mapping:humble \

bash -lc 'source /opt/ros/humble/setup.bash && source /root/yahboomcar_ws/install/setup.bash && export PYTHONPATH=/workspace/ros2/dogzilla_slam:$PYTHONPATH && python3 -m pytest -q -p no:cacheprovider ros2/dogzilla_slam/test deploy/test'

sudo grants access to the Docker daemon. docker run creates a one-off container. --rm removes it after the test, --network none prevents network access, --volume ...:/workspace:ro mounts the current source read-only and --workdir /workspace selects its working directory. The next value is the local image. bash -lc runs the quoted configured shell commands. The two source commands load ROS Humble and the built workspace. PYTHONPATH places the mounted current package ahead of the image's older installed copy. python3 -m pytest runs the test runner, -q shortens output and -p no:cacheprovider prevents writes to the read-only repository. The final paths select the package and deployment tests. No --device flag is supplied, so this test cannot open the robot controller, LiDAR or camera.

The deployment coordinator's dependency-light subset can also run on the host:

python3 -m unittest discover -s deploy/test -p 'test_*.py'

python3 -m unittest discover uses the standard-library test discovery. -s sets the starting directory and -p sets the filename pattern. This does not replace the complete ROS-container suite.

Check formatting damage and unresolved merge text before a build:

git diff --check

grep -RInE '^(<<<<<<<|=======|>>>>>>>)' ros2 deploy docs

git diff --check detects whitespace errors in tracked changes. grep searches recursively; -R recurses, -I ignores binary files, -n prints line numbers, and -E enables the conflict-marker expression.

Then verify the host and build:

dogzilla doctor

dogzilla build

doctor checks Docker, Compose, serial devices, the pinned base image, display and free storage. build creates the local dogzilla-mapping:humble image; it does not deploy a mode, push Git, or upload an image.

Use the shortest physical test that covers the change. A web validation change normally needs API/unit tests and a stationary mission start before motion. A controller change needs fake-controller tests, stationary telemetry, then a slow open-floor trial.

## Configuration sources of truth

| Concern | Source |
| --- | --- |
| Controller speed levels | dogzilla_slam/speed_control.py |
| Controller clamps/watchdog/battery | dogzilla_slam/safe_base.py |
| Mapping behavior | config/dogzilla_2d.lua or dogzilla_2d_imu.lua |
| Localization behavior | config/dogzilla_localization.lua or IMU variant |
| Nav2 controller/costmaps/footprint | config/nav2_test1.yaml |
| Nav2 recovery policy | behavior_trees/*.xml |
| Velocity arbitration | config/twist_mux.yaml |
| Camera profile | config/mono_camera.yaml |
| Vision detection policy | vision_core.py, object_detector.py, vision_node.py |
| Firmware action allowlist | vision_action_policy.py |
| Web validation/state/database | web_core.py |
| ROS and task orchestration | web_gateway.py |
| HTTP routes/authentication | web_http.py |
| Container/device boundary | deploy/compose.yaml |
| Host lifecycle and safety checks | deploy/dogzilla-map, deploy/dogzilla-mission |

Avoid duplicating a number in another layer unless the layer has a genuinely different physical meaning. When duplication is required, add a contract test that detects divergence.

## Mapping and localization model

Each operational map is a three-file bundle:

- .pbstream: Cartographer frozen trajectory, submaps and pose-graph state;

- .pgm: occupancy pixels;

- .yaml: Nav2 resolution, origin and occupancy metadata.

Mapping generates all three. Localization loads PBStream as frozen state and starts a live trajectory relative to it. The map server publishes PGM/YAML for Nav2 and the web.

The normal mission start waits for an operator initial pose. The web gateway uses that pose as a search centre, evaluates nearby position/angle candidates, publishes the best accepted /initialpose, then checks stable TF, correction size and live scan/map agreement. --match is an explicit alternative that starts Cartographer's global matching without the operator pose gate.

Furniture can reduce endpoint agreement or create ray contradictions. The validator is evidence of geometric consistency, not a proof that every saved wall is currently visible. Threshold changes require recorded comparison data, not one convenient room trial.

## Navigation model

The global planner uses the static map, live LiDAR obstacle layer, inflation and the active map's keepout mask. The regulated pure-pursuit controller follows the path using a local costmap. The velocity smoother and steering guard reduce abrupt command changes before Twist Mux and safe_base.

The default measured footprint is approximately 260 x 145 mm with 30 mm padding. Automatic spin and backup recovery are disabled because they are high risk on the quadruped and were not physically accepted.

The tuning recorder captures only data needed to diagnose navigation: planned paths, raw/smoothed/final commands, scan odometry, map/odom poses, cross-track error, LiDAR sectors, timing, diagnostics, result and parameter changes. Use this evidence before changing controller gains.

## Web task model

The SQLite store persists deliveries, routes, patrols, named locations, map-specific keepouts, hazard observations and vision alerts. On gateway restart, any task that was active is marked failed; movement never silently resumes.

Task flow:

queued -> running -------------------------------------> completed

|  |

|  +-> pausing -> paused ------------------+

|  +------------> waiting -----------------+-> running

+-> cancelling ---------------------------> cancelled

`------------------------------------------> failed

Only one task is active. waiting for manual continuation is still active and blocks the next queued task. Pause/continue and manual checkpoints apply to the one-to-ten-stop delivery/waypoint-mission type; the generic route and patrol types do not inherit those controls automatically.

Patrol adds a perception gate. It will remain queued unless Vision reports mode patrol, complete model coverage, multi-frame confirmation settings and action_output: disabled. At present the operator must select Patrol vision; queuing a patrol does not automatically change the mode.

## Persistent and generated data

Git intentionally ignores credentials, runtime logs, task databases, captured profiles and local build products. A Git backup alone is not a complete robot backup.

Back up these robot-specific assets separately:

- maps/ map bundles;

- calibration/imu.json;

- real calibration/camera.yaml and measured camera extrinsics;

- web/data/tasks.sqlite3 if mission history/locations matter;

- selected log sessions and validation reports;

- any reviewed model weights not reproducible from documented downloads;

- controller firmware version and physical measurements.

Never add API keys, .env, browser passwords or raw private datasets to Git.

## Logging and diagnosis

Every mode creates a timestamped directory under logs/sessions/ and updates logs/latest. Mission Mode keeps navigation, perception and web in the same logical session.

Begin diagnosis with:

- operator status;

- the exact task state/error;

- localization state and live scan rate;

- Nav2 action availability;

- controller/battery state;

- container logs from the same session;

- tuning report for motion-quality problems.

Do not tune Nav2 when the underlying pose or scan timestamps are unstable. Do not tune Cartographer to hide a wrong initial pose. Do not add more speed to compensate for a controller oscillation.

## Safe change ownership

| Change | Primary files | Minimum verification |
| --- | --- | --- |
| Browser layout | web_static/* | dashboard tests, desktop/mobile browser check |
| API validation/task state | web_core.py, web_gateway.py, web_http.py | unit/API tests, stationary mission state check |
| Patrol/object classes | vision modules, model labels | model validation, camera-only acceptance, patrol gate test |
| Nav2 tuning | nav2_test1.yaml, behavior trees, steering guard | contract tests, stationary timing, recorded open-floor trial |
| Cartographer tuning | localization/mapping Lua | scan/TF timing, known-map replay or controlled trial |
| IMU correction | calibration and IMU modules | six-pose calibration and imu-check |
| Controller behavior | safe_base.py, policy modules | firmware doc review, fake controller, battery gate, physical safety trial |
| Docker dependencies | Dockerfile and package lock | clean image build, package discovery, health checks |
| URDF/camera transform | Xacro and measured calibration | physical measurement, TF validation; no control use |

## Current limitations

- Embedded controller firmware source is unavailable.

- Firmware-identical rest/torque release is not implemented.

- Operational odometry depends on LiDAR scan matching.

- IMU fusion is optional and has not improved every route.

- Monocular vision does not provide dependable depth by itself.

- RTAB shadow is not operational navigation authority.

- URDF leg geometry and joint conventions remain provisional.

- No physical bumper/contact sensor confirms a collision.

- Patrol vision mode does not yet switch automatically with a patrol task.

- Navigation stability candidate settings are prepared but not automatically active; inspect plans/navigation-stability/README.md before using them.

## Handoff checklist

Before accepting responsibility for the robot:

- read the seven documents listed at the top;

- confirm the physical E-stop/power procedure with the owner;

- run dogzilla doctor;

- record the installed DOGZILLALib and controller firmware versions;

- verify the current Git branch and dirty files;

- confirm which Docker image ID is deployed;

- make a robot-specific backup of maps and calibration;

- perform a stationary status check;

- run teleop level 1 on a clear floor before autonomous testing;

- validate initial pose and one short waypoint before patrol/delivery trials;

- do not enable rest replay, URDF control or aggressive recoveries without a new physical acceptance plan.

Source chapter: docs/FRAMEWORK.md

# DOGZILLA S2 software framework

This document describes the software currently implemented in this repository. It separates the operational robot stack from experimental components so that future work does not accidentally bypass a hardware or safety boundary.

## System boundary

Raspberry Pi host

|

+-- operator commands: deploy/dogzilla-map and deploy/dogzilla-mission

+-- persistent data: maps, calibration, profiles, logs, web/data

`-- Docker Compose (ROS 2 Humble, ROS_DOMAIN_ID 12)

|

+-- navigation OR mapping OR drive       owns robot serial devices

+-- perception                           owns the mission camera

`-- web                                  owns no hardware

Normal Mission Mode is three cooperating containers:

| Container | Responsibility | Hardware access |
| --- | --- | --- |
| dogzilla_navigation | LiDAR, controller bridge, localization, Nav2, command filtering | /dev/ttyAMA0, /dev/ttyAMA1 |
| dogzilla_perception | Camera capture and detection-only vision | /dev/video0 |
| dogzilla_web | Browser UI, validation, task execution and persistence | None |

All three use host networking and the same ROS domain. Only the navigation container can move the robot. The web and perception containers cannot open the leg controller serial port.

## Hardware ownership

| Device | Owner | Data or action |
| --- | --- | --- |
| /dev/ttyAMA0 | dogzilla_safe_base only | Movement, stop, battery, 12 joint angles and optional raw IMU |
| /dev/ttyAMA1 | MS200 driver | 2-D LaserScan on /scan |
| /dev/video0 | One camera service at a time | 640 x 480 mono-camera frames |

The single-owner rule for /dev/ttyAMA0 is mandatory. Yahboom programs, a second safe_base, or a separate IMU reader must not run beside an operational mode. Competing processes can split controller replies and make movement and telemetry unreliable.

safe_base is the hardware safety boundary. It:

- stops the controller during startup and shutdown;

- clamps linear and angular velocity;

- uses independent movement and turning levels from 1 through 9;

- stops after a 0.6 second command timeout;

- accepts the latched /safety/estop signal;

- enforces the controller low-battery movement lockout;

- publishes /battery_state, /joint_states, and optional raw IMU data.

## Coordinate frames

map

`-- odom                         Cartographer localization correction

`-- base_link                scan-matched robot motion

+-- laser_frame          x=0.000, y=0.000, z=0.180 m

+-- imu_link             x=0.085, y=0.000, z=0.070 m

`-- camera_link

`-- camera_optical_frame

map is the saved-map coordinate system used by waypoints and keepout zones. odom is the continuous local motion frame. base_link is the robot body reference used by Cartographer and Nav2. The camera transform is valid only after its extrinsics are physically measured.

The URDF under ros2/dogzilla_slam/urdf/ is a visualization framework. Leg dimensions, axes, corner assignments, offsets, and limits remain provisional; it must not be used for control or collision checking yet.

## ROS components

### Hardware and state estimation

| Component | Role |
| --- | --- |
| oradar_lidar | Publishes MS200 scans on /scan and uses the patched motor-off shutdown path. |
| dogzilla_safe_base | Converts /cmd_vel into guarded controller commands and publishes controller telemetry. |
| dogzilla_imu_corrector | Applies axis, gravity, bias, scale and covariance calibration to raw IMU data. |
| Cartographer | Builds maps or localizes against a frozen PBStream. |
| dogzilla_localization_manager | Starts, replaces, pauses and cancels Cartographer localization trajectories from /initialpose. |
| dogzilla_tf_odometry | Converts odom -> base_link scan-matched TF into /odom for Nav2. |

DOGZILLA has no wheel encoders. Its operational odometry is derived from Cartographer scan matching. IMU fusion is optional and should be enabled only with a validated calibration/imu.json.

### Navigation and motion commands

Nav2 controller

-> /cmd_vel_nav_raw

-> velocity_smoother

-> /cmd_vel_nav_smoothed

-> dogzilla_steering_guard

-> /cmd_vel_nav

-> twist_mux ---------------------> /cmd_vel -> dogzilla_safe_base

keyboard / web safety stop

-> /cmd_vel_teleop ---------------^

The keyboard channel has higher priority than Nav2. The steering guard rejects rapid left/right reversals without changing the requested forward motion. Nav2 uses a regulated pure-pursuit controller, the measured 260 x 145 mm body footprint plus padding, live LiDAR obstacle layers, static-map obstacles and per-map keepout masks. Automatic spin and backup recovery are disabled.

Navigation diagnostics and the tuning recorder are observation-only. They can write bounded reports, but cannot command, cancel or slow the robot.

### Web mission control

dogzilla_web_gateway connects the browser to ROS. It owns:

- password-authenticated HTTP and event-stream endpoints;

- live map, pose, battery, joints, vision and task state;

- initial-pose search and LiDAR-to-map verification;

- map-specific named locations, patrol areas and keepout zones;

- route preview through Nav2 without movement;

- one-at-a-time delivery, waypoint-route and patrol execution;

- autonomy speed/turn settings and the software emergency stop;

- SQLite task history and the last 25 vision alerts/photos.

The gateway validates every goal against its own occupancy grid. Browser-side checks are for feedback only and are not trusted as the safety decision.

Task states are:

queued -> running -------------------------------------> completed

|  |

|  +-> pausing -> paused ------------------+

|  +------------> waiting -----------------+-> running

+-> cancelling ---------------------------> cancelled

`------------------------------------------> failed

Only one task can be active. A task in waiting for manual continuation still owns the executor, so a queued patrol will not move until that task is continued or cancelled. Pause/continue and manual checkpoints belong to the dashboard's one-to-ten-stop waypoint mission, stored internally as task kind delivery.

### Vision and patrol

The perception container publishes:

| Topic | Contents |
| --- | --- |
| /vision/status | Active mode, detector readiness and non-actuating status |
| /vision/detections | Structured detections |
| /vision/danger_confirmed | Multi-frame confirmed person or hazard event |
| /vision/annotated/compressed | Browser image with overlays |
| /vision/mode_command | Web-to-perception configuration request |

Normal Mission Mode keeps vision action output disabled. A confirmed hazard can create a notification and photo, but it does not stop an autonomous task. Duplicate alerts are suppressed within the configured cooldown and stored photos are capped at 25.

Autonomous patrol is stricter than ordinary delivery. It requires:

- a saved patrol polygon on the active map;

- at least two generated safe coverage waypoints;

- verified localization, Nav2, fresh pose, map and acceptable battery;

- vision mode patrol, complete detector coverage and disabled action output;

- no other active or waiting task.

At present, queuing a patrol does not automatically change the vision mode. The operator must apply Patrol in the Vision panel before the patrol task can leave queued.

## Persistence

| Path | Persistent content |
| --- | --- |
| maps/ | PBStream, PGM and YAML map bundles |
| calibration/ | IMU calibration, camera intrinsics and extrinsics |
| profiles/ | Captured controller profiles; firmware-rest replay remains disabled |
| logs/sessions/ | Timestamped ROS and diagnostic sessions |
| logs/latest | Symlink to the newest session |
| web/data/tasks.sqlite3 | Tasks, locations, patrol areas, keepouts, hazards and alerts |
| web/data/alerts/ | Bounded alert photographs |

Containers are disposable; these host-mounted paths are the durable state. Docker images are build products and are not stored in Git.

## Operational status

| Area | Status |
| --- | --- |
| LiDAR-only mapping and map saving | Operational |
| Initial-pose localization and wide nearby search | Operational, requires operator confirmation |
| Nav2 waypoint, delivery and route missions | Operational, supervised development use |
| Patrol route generation and detection gate | Implemented; vision mode must currently be selected manually |
| IMU correction | Implemented and calibrated; optional because it has not improved every navigation trial |
| URDF visualization | Experimental; measurements incomplete |
| Monocular RTAB-Map shadow | Experimental, calibration-gated and non-controlling |
| Firmware-identical rest/release | Not implemented; replay intentionally disabled |

## Source map

| Path | Purpose |
| --- | --- |
| deploy/dogzilla-map | Main host operator command |
| deploy/dogzilla-mission | Mission startup, health checks, switching and rollback |
| deploy/compose.yaml | Container boundaries, devices and persistent mounts |
| deploy/Dockerfile | Reproducible ARM64 ROS image |
| ros2/dogzilla_slam/launch/ | Combined ROS launch descriptions |
| ros2/dogzilla_slam/config/ | Cartographer, Nav2, Twist Mux, camera and RTAB parameters |
| ros2/dogzilla_slam/dogzilla_slam/ | Robot, localization, navigation, vision and web nodes |
| ros2/dogzilla_slam/behavior_trees/ | Fail-closed Nav2 behavior trees |
| ros2/dogzilla_slam/urdf/ | Provisional robot-description framework |
| ros2/dogzilla_slam/test/ | Pure-Python and deployment contract tests |

Source chapter: docs/PIPELINES.md

# DOGZILLA S2 pipelines

These pipelines show how sensor data, operator requests and robot commands move through the implemented system. The arrows describe runtime ownership, not just which files import one another.

For presentations, use the simplified four-layer 16:9 diagram as either scalable SVG or a ready-to-insert 1920 x 1080 PNG.

## 1. Mapping pipeline

MS200 LiDAR (/dev/ttyAMA1)

-> oradar driver

-> /scan

-> Cartographer online scan matching

-> map -> odom -> base_link TF

-> Cartographer occupancy-grid node

-> /map (default 0.05 m cells)

-> RViz and map saver

-> MAP.pbstream + MAP.pgm + MAP.yaml

Movement during mapping follows a separate guarded path:

keyboard teleop -> /cmd_vel -> dogzilla_safe_base -> /dev/ttyAMA0

The PBStream preserves Cartographer's pose graph and scan data for future pure localization. PGM/YAML preserve the 2-D occupancy grid used by Nav2 and the web map. Both forms belong to one map bundle and should be kept together.

Optional IMU path:

/dev/ttyAMA0

-> dogzilla_safe_base

-> /imu/data_uncalibrated

-> dogzilla_imu_corrector + calibration/imu.json

-> /imu/data_corrected

-> Cartographer IMU configuration

## 2. Localization pipeline

saved MAP.pbstream --------------------------> frozen Cartographer trajectory

saved MAP.yaml + MAP.pgm --------------------> /map

operator estimated pose + current /scan

-> web coarse-to-fine nearby search

(configured translation and angle window)

-> candidate score and ambiguity checks

-> /initialpose

-> localization_manager starts live trajectory relative to frozen map

-> Cartographer scan matching

-> map -> odom -> base_link

-> web stability + scan/map agreement checks

-> localization state READY

The initial pose is an estimated search centre, not a claim that the exact pose is already known. Mission dispatch remains blocked until the resulting pose is stable and the live scan agrees with the saved occupancy map. The operator can stop matching and submit a new initial pose without restarting all containers.

## 3. Navigation pipeline

validated map-frame goal

-> Nav2 planner + global costmap

- static map

- live /scan obstacles

- map-specific keepout mask

-> global path

-> regulated pure-pursuit controller + local costmap

-> /cmd_vel_nav_raw

-> velocity smoother

-> /cmd_vel_nav_smoothed

-> steering guard

-> /cmd_vel_nav

-> Twist Mux

-> /cmd_vel

-> dogzilla_safe_base clamp + watchdog

-> controller serial

-> leg movement

Motion feedback closes the loop through Cartographer TF and /odom. There is no wheel-encoder feedback. If scan matching is unstable, Nav2 receives unstable pose and heading estimates even when its planned path is straight.

## 4. Web mission pipeline

browser map click

-> browser preview validation

-> authenticated web API

-> authoritative occupancy + keepout validation

-> Nav2 ComputePathToPose preview (no movement)

-> SQLite task in QUEUED state

-> runtime gate

- E-stop clear

- selected map matches

- localization verified

- Nav2 action available

- map and pose fresh

- fresh, valid battery telemetry at or above the task threshold

-> one NavigateToPose action per waypoint

-> result + progress persisted

-> browser event stream update

The dashboard's one-to-ten-stop waypoint mission (internally task kind delivery) may pause or wait at a waypoint:

waypoint reached

+-> automatic: wait configured time -> next waypoint

`-> manual: task WAITING -> operator Continue -> next waypoint

A waiting task remains active. New tasks stay queued until it is continued or cancelled.

Invalid or stale battery telemetry blocks the start of a new task. Once a task is moving, a single invalid or stale reading does not stop it; only a confirmed valid reading below the configured threshold triggers the active-task safety path.

## 5. Patrol pipeline

draw polygon on active map

-> validate bounds, area and self-intersection

-> save patrol area for that map

-> generate serpentine coverage points

-> reject occupied, unknown, boundary and keepout points

-> preview route

-> queue patrol task

-> patrol-specific readiness gate

-> send coverage waypoints to Nav2 one at a time

-> repeat configured number of cycles

-> complete task

The patrol-specific gate adds the following vision pipeline:

/dev/video0

-> usb_cam

-> /camera/image_raw

-> dogzilla_vision in PATROL mode

+-> person and face-presence detection

+-> dangerous-object models

+-> multi-frame confidence / IoU confirmation

-> /vision/status must report ready + action_output disabled

-> /vision/danger_confirmed

-> web notification + annotated photo + robot map pose

-> duplicate cooldown and last-25 retention

Vision is observational during patrol. It reports hazards but does not directly move or stop the robot. The current UI requires the operator to apply Patrol vision mode before a queued patrol can start. Selecting Patrol vision alone does not create a movement task; a saved area must also be queued.

## 6. Keepout pipeline

polygon drawn on map

-> server validation

-> SQLite record keyed by map name

-> rasterized OccupancyGrid mask

-> /keepout_filter_mask + /keepout_filter_info

-> Nav2 global and local costmap filters

-> planning and control avoid the zone

Keepout zones do not leak between maps. Switching maps loads and publishes only the zones associated with the new map.

## 7. Camera and RTAB-Map shadow pipeline

/dev/video0

-> usb_cam

-> rectification using measured camera intrinsics

-> /camera/image_rect + /camera/camera_info

Cartographer odom -> base TF

-> isolated /rtabmap_shadow/odom_input

rectified mono + CameraInfo + /scan + isolated odom

-> namespaced RTAB-Map

-> /rtabmap_shadow/* + persistent shadow database

Shadow mode publishes no movement command and no competing TF. It is an experimental visual-memory pipeline, not the authority for current Nav2 localization. It remains gated on real camera intrinsics and measured camera extrinsics.

## 8. Build and deployment pipeline

repository source

-> unit and contract tests

-> deploy/Dockerfile

- pinned Yahboom base image

- locked ROS packages

- checksum-pinned usb_cam fix

- patched MS200 motor shutdown

- colcon build of usb_cam, oradar_lidar and dogzilla_slam

-> local dogzilla-mapping:humble image

-> Docker Compose services

-> health checks

-> timestamped ROS log session

Normal release sequence:

doctor -> test -> build -> start requested mode -> verify health -> supervised trial

deploy/dogzilla-map build replaces the local image only. It does not push to GitHub or a Docker registry. Source backup and runtime deployment are separate:

- Git stores code, configuration and documentation.

- Docker stores the locally built runtime image.

- Host mounts store maps, calibration, databases, alerts and logs.

## Failure boundaries

| Failure | Expected result |
| --- | --- |
| LiDAR or map unavailable | Mapping/navigation health check fails; mission does not dispatch. |
| Initial pose not verified | Autonomous task remains queued. |
| Another task is waiting | New patrol/delivery remains queued. |
| Patrol vision is raw or incomplete | Patrol remains queued; ordinary delivery may still run. |
| Nav2 aborts a waypoint | Task becomes failed with the action status in its error. |
| Software E-stop | Active goal is cancelled, zero velocity is published and the latch must be reset. |
| Web restart during an active task | Interrupted task is marked failed rather than silently resumed. |
| Container shutdown | safe_base stops motion and the patched LiDAR driver stops the motor. |

Figure 1. Simplified four-layer DOGZILLA pipeline for presentations.

Source chapter: docs/FIRMWARE_AND_SERIAL.md

# DOGZILLA controller firmware and serial interface

This document explains the controller boundary used by this repository. It is intended for developers who need to extend DOGZILLA without creating a second serial owner or assuming undocumented firmware behavior.

## What “firmware” means on this robot

DOGZILLA has four distinct software layers:

- Embedded controller firmware runs on the motor controller. It performs gait generation, inverse kinematics, servo control, action sequences, battery protection and the physical power-button behavior.

- Yahboom DOGZILLALib runs on the Raspberry Pi. It converts Python method calls into controller serial packets.

- The original Yahboom applications provide the mobile-app server, joystick, camera lessons and OLED display.

- This repository's ROS 2 layer provides single-owner serial access, command limits, telemetry, mapping, navigation, vision and web missions.

Only layers 2 through 4 are visible as source on this Pi. The embedded controller source and action trajectories are not in this repository. The controller can therefore be documented only from its public host protocol, readbacks and observed physical behavior.

## Versions present on this Pi

| Component | Version or status |
| --- | --- |
| Installed host library | DOGZILLALib 3.1.9, modified 2023-05-23 |
| Historical repository archive | DOGZILLALib.zip contains 2.0.7 |
| Original app header | Reports V2.0.7 |
| Embedded firmware binary/source | Not stored in this repository |
| Embedded version | Queryable with read_version() when the serial port is free |

The Docker image inherits the host library from the pinned Yahboom base image. Do not replace it with the older archive merely because the archive is inside the repository: 3.1.9 adds read_imu_raw() and has different timing behavior.

To inspect the installed host-library version without opening the controller:

python3 -c "import DOGZILLALib.DOGZILLALib as d; print(d.__version__)"

python3 runs the host Python interpreter. -c executes the quoted one-line program. The program imports the implementation module and prints its declared version; it does not instantiate DOGZILLA and therefore does not open /dev/ttyAMA0.

## Physical connection and ownership

The host library opens:

/dev/ttyAMA0, 115200 baud, 8 data bits, no parity, one stop bit

Only one process may own this port. Read operations flush the input buffer before sending a request, so two readers do not merely share bandwidth: each can erase the other process's reply and misassociate subsequent packets.

During ROS modes, dogzilla_safe_base is the only permitted owner. The operator wrapper:

- gracefully stops the original app_dogzilla.py process;

- pauses yahboom_oled.service, which otherwise opens its own controller;

- verifies that the serial devices are free;

- starts one Compose service with /dev/ttyAMA0 passed through;

- restores the OLED service after the ROS owner stops.

Do not run the original notebooks, OLED script, joystick script, a standalone IMU reader, or an interactive DOGZILLALib shell while mapping, navigation, drive or armed vision control is active.

## Serial frame format

The host library uses a small binary register protocol.

### Write packet

55 00 LEN 01 ADDR DATA... CHECK 00 AA

LEN is the full packet length, equal to the payload length plus 8. 01 is the write type. ADDR is the register address. CHECK is:

255 - ((LEN + TYPE + ADDR + sum(DATA)) modulo 256)

### Read request

55 00 09 02 ADDR READ_LEN CHECK 00 AA

02 is the read type and READ_LEN is the requested payload size. The reply uses the same 55 00 ... 00 AA envelope. The library validates the length and checksum before exposing values.

This protocol has no transaction identifier and the sensor packet has no hardware timestamp. That is another reason one process must serialize every request and reply.

## Public register map

The following addresses come from installed DOGZILLALib 3.1.9. A range means one address per axis, leg coordinate or motor.

| Address | Library name | Purpose |
| --- | --- | --- |
| 0x01 | BATTERY | Read integer battery percentage |
| 0x03 | PERFORM | Enable or disable repeating performance mode |
| 0x04 | CALIBRATION | Enter or leave software calibration |
| 0x05 | UPGRADE | Firmware upgrade register; not used here |
| 0x06 | MOVE_TEST | Vendor movement test register; not used here |
| 0x07 | FIRMWARE_VERSION | Read controller firmware string |
| 0x09 | GAIT_TYPE | Trot, walk or high-walk selection |
| 0x13 | BT_NAME | Bluetooth-name storage in the public map |
| 0x20 | LOAD/UNLOAD_MOTOR | Torque/load operation selected by payload |
| 0x30 | VX | Forward/backward command |
| 0x31 | VY | Lateral command |
| 0x32 | VYAW | Turning command |
| 0x33–0x35 | TRANSLATION | Body x, y and z translation |
| 0x36–0x38 | ATTITUDE | Body roll, pitch and yaw |
| 0x39–0x3B | PERIODIC_ROT | Periodic roll, pitch and yaw |
| 0x3C | MarkTime | Stationary stepping height |
| 0x3D | MOVE_MODE | Slow, normal or high pace |
| 0x3E | ACTION | Run a firmware action group |
| 0x40–0x4B | LEG_POS | Three coordinates for each of four legs |
| 0x50–0x5B | MOTOR_ANGLE | Command or read twelve servo angles |
| 0x5C | MOTOR_SPEED | Speed for direct servo-angle control |
| 0x61 | IMU | Enable or disable controller stabilization |
| 0x62–0x64 | ROLL/PITCH/YAW | Read fused controller attitude |
| 0x65 | IMU_RAW | Read 24-byte acceleration, gyro and RPY payload |
| 0x80–0x82 | PERIODIC_TRAN | Periodic x, y and z translation |

Registers described by the library are not automatically safe to expose in ROS. Upgrade, calibration, direct motor, load/unload and high-walk functions require separate physical test plans.

## Host-library scaling and limits

The library maps physical-looking values into unsigned bytes. Values outside the configured range are saturated.

| Command | Public range or wrapper behavior |
| --- | --- |
| move_x() | Clamped to -20 through 20; protocol scale limit is 25 |
| move_y() | Clamped to -18 through 18 |
| turn() | Clamped to -70 through 70; non-zero magnitudes below 30 become 30 |
| Body translation x/y/z | ±35, ±19.5, and 75–115 respectively |
| Body roll/pitch/yaw | ±20, ±22 and ±16 respectively |
| Mark-time height | 10–35 when non-zero |
| Direct motor speed | 1–255; input zero is converted to 1 |
| Motor IDs | 11–13, 21–23, 31–33 and 41–43 |

The controller command values are not SI velocities. safe_base provides a ROS-facing scale and clamps /cmd_vel, but DOGZILLA has no wheel encoder that proves commanded metres per second. Scan-matched odometry is the measured motion source used by navigation.

turn() has a firmware-library dead zone: any non-zero command becomes at least 30 controller units. The separate 1–9 turn profile and steering guard exist partly to manage this discontinuity.

## Public Python API by function

| Group | Methods |
| --- | --- |
| Velocity | move, move_x, move_y, forward, back, left, right, turn, turnleft, turnright, stop |
| Body pose | translation, attitude, periodic_tran, periodic_rot, mark_time |
| Gait/mode | pace, gait_type, imu, perform |
| Preset actions | action, reset |
| Direct limbs | leg, motor, motor_speed |
| Torque | load_motor, load_allmotor, unload_motor, unload_allmotor |
| Telemetry | read_battery, read_motor, read_version, read_roll, read_pitch, read_yaw, read_imu_raw |
| Maintenance | calibration |

stop() writes zero x velocity, zero y velocity, zero mark-time command and zero yaw command. It does not unload the servos or power down the controller.

reset() sends action ID 255. The stock application treats this as stop and return to its initial posture.

## Preset action groups

The fixed action names below are the public mapping used by Yahboom samples and the guarded vision policy.

| ID | Name | ID | Name |
| --- | --- | --- | --- |
| 1 | lie down | 11 | pee |
| 2 | stand up | 12 | sit down |
| 3 | crawl | 13 | wave hand |
| 4 | turn around | 14 | stretch |
| 5 | mark time | 15 | wave body |
| 6 | squat | 16 | swing |
| 7 | turn roll | 17 | pray |
| 8 | turn pitch | 18 | seek |
| 9 | turn yaw | 19 | handshake |
| 10 | three axis | 255 | reset/default posture |

The stock Raspberry Pi app sends motor speed 50 and action 14 at startup as its stretch display. This is an application behavior, not evidence that the embedded controller automatically performs action 14 on every power-up.

Action IDs identify firmware-owned trajectories, but their joint curves, timing, torque transitions and completion acknowledgements are not published. The host sends a command and normally waits a conservative guard time.

## IMU packet behavior

read_imu_raw() requests 24 bytes and returns nine values:

accX, accY, accZ, gyroX, gyroY, gyroZ, roll, pitch, yaw

The host library converts signed 16-bit acceleration using 9.8 / 16384 and gyro using 1 / 16.4. Gyro values are degrees per second. The last three values are controller-provided floats converted toward radians.

The ROS boundary intentionally uses only raw acceleration and gyro:

- gyro is converted from degrees/s to radians/s;

- the message is timestamped after the complete packet arrives because the packet has no sensor clock;

- controller orientation is marked unavailable because its world-frame and fusion convention are undocumented;

- imu_corrector applies the robot-specific axis mapping, gravity convention, scale, gyro bias and measured covariance from calibration/imu.json.

## Battery and low-power behavior

The public readback is an integer percentage. It does not expose voltage, current, cell balance, temperature, remaining capacity or a hardware timestamp. A zero value means the read failed; it is not a confirmed empty battery.

Observed/controller behavior and repository behavior are deliberately split:

| Condition | Embedded controller | ROS safe_base |
| --- | --- | --- |
| Valid battery above 25% | Normal controller behavior | Movement permitted unless E-stop is latched |
| Valid battery at or below 25% | Controller's private low-battery rest is expected to win | Sends stop, inhibits ROS movement and waits for recovery to at least 28% |
| Invalid battery read | Unknown; no new fact is available | Retains the last confirmed safety state; does not treat zero as a real 0% reading |
| New autonomous task | No task concept | Web requires a fresh valid reading at or above its configured task threshold, default 28% |
| Active autonomous task | No task concept | A stale/invalid reading alone does not stop motion; a confirmed valid low reading does |

The physical power-button shutdown and low-battery rest appear smooth because they are controller-owned routines. Their exact joint sequence and torque-off timing are not exposed by DOGZILLALib.

## Rest, stand and servo release

These operations are not interchangeable:

- Action 1 is the public lie-down animation.

- unload_allmotor() immediately requests servo release.

- The power-button/low-battery routine is a private safety trajectory followed by controller-managed torque behavior.

This repository does not claim action 1 plus unload is equivalent to the private routine. dogzilla rest is therefore disabled and sends no movement or torque command. This protects the robot from an abrupt fall or joint collision.

dogzilla stand is supported with a narrow path:

- require a valid battery strictly above 25%;

- send controller stop;

- load all motors at their current positions;

- send public stand-up action 2;

- hold a four-second action guard before closing the port.

The passive rest recorder reads only battery and twelve motor angles. It can capture a naturally occurring low-battery trajectory with pre-roll and a stationary tail. Every capture is marked replay_enabled: false; a captured curve is evidence, not an approved command profile.

## Original Yahboom application

app_dogzilla/app_dogzilla.py is the stock-style mobile-app server. It:

- opens DOGZILLALib directly;

- hosts a TCP/app and camera interface;

- creates joystick and OLED helpers around the controller object;

- sets motor speed 50 and plays stretch action 14 at startup.

Standalone joystick_dogzilla.py and oled_dogzilla.py also open the serial port directly. The provided systemd OLED service is therefore incompatible with a simultaneous ROS serial owner. The deployment wrapper handles this service explicitly.

The original kill_dogzilla.sh uses process matching and kill -9. The new operator wrapper does not use that as its normal path: it first requests Ctrl-C and refuses to take the serial port if the vendor application does not close cleanly.

## Extending controller support safely

When adding a controller feature:

- Add it to dogzilla_safe_base or another component inside the same serial-owning process. Do not create a second /dev/ttyAMA0 node.

- Validate every parameter before opening the port.

- Stop walking before posture, mode, action, direct motor or torque changes.

- Require a valid battery for any new action that can move or load servos.

- Use bounded serial timeouts. The project wraps the vendor's private one-second read loop down to a configured maximum of 0.08 seconds.

- Publish explicit state so the web UI and logs show whether a request was accepted, blocked or timed out.

- Add fake-controller unit tests for packet-independent policy and failure handling.

- Test one new physical operation at a time on a clear, level floor with an operator ready to use the physical power control.

Do not introduce raw register writes into the browser, vision node or mission executor. Those components are intentionally hardware-free.

## Unknowns that must stay labelled unknown

The following cannot be reconstructed from the public Python library alone:

- embedded inverse kinematics and gait phase logic;

- servo PID gains and current/torque limits;

- action trajectory curves and completion state;

- the battery percentage estimator;

- the physical power-button state machine;

- low-battery rest and servo-release timing;

- controller self-stabilization coordinate conventions;

- whether Bluetooth control arbitrates with serial commands internally;

- firmware upgrade format and recovery behavior.

Do not fill these gaps with guessed motor sequences. Obtain official controller documentation, capture read-only evidence, or build a separately reviewed physical test before changing the safety boundary.

Source chapter: docs/ROS_INTERFACES.md

# DOGZILLA ROS 2 interface reference

This is the developer-facing contract between hardware, mapping, localization, navigation, vision and the web gateway. Default ROS domain ID is 12.

## Frames

map

`-- odom

`-- base_link

+-- laser_frame

+-- imu_link

`-- camera_link

`-- camera_optical_frame

| Frame | Owner and meaning |
| --- | --- |
| map | Saved-map coordinate system. Cartographer localization supplies the correction to odom. Web waypoints and keepouts are expressed here. |
| odom | Continuous local scan-matched frame. It is not wheel odometry. |
| base_link | Robot body reference used by Cartographer and Nav2. |
| laser_frame | MS200 scan origin, currently 0.18 m above base_link. |
| imu_link | Calibrated IMU frame, currently (0.085, 0, 0.070) m from base_link. |
| camera_link | Physical camera mount; operational only after measured extrinsics. |
| camera_optical_frame | REP-103 optical convention: z forward, x right, y down. |

Operational Cartographer owns map -> odom -> base_link. RTAB shadow mode publishes no competing TF. The experimental URDF must not become a second owner of existing LiDAR or IMU transforms.

## Core topics

### Hardware and state estimation

| Topic | Type | Publisher | Main consumers |
| --- | --- | --- | --- |
| /scan | sensor_msgs/msg/LaserScan | MS200 driver | Cartographer, Nav2 costmaps, web pose validation, RTAB shadow |
| /battery_state | sensor_msgs/msg/BatteryState | dogzilla_safe_base | Web task gate and operator telemetry |
| /joint_states | sensor_msgs/msg/JointState | dogzilla_safe_base | Web telemetry, URDF visualization, passive rest capture |
| /imu/data_uncalibrated | sensor_msgs/msg/Imu | dogzilla_safe_base when enabled | dogzilla_imu_corrector |
| /imu/data_corrected | sensor_msgs/msg/Imu | dogzilla_imu_corrector | IMU Cartographer profiles |
| /map | nav_msgs/msg/OccupancyGrid | Cartographer occupancy grid in mapping; Nav2 map server in localization | RViz, Nav2, web gateway |
| /odom | nav_msgs/msg/Odometry | dogzilla_tf_odometry | Nav2 controller, velocity smoother, diagnostics, tuning recorder, web |
| /initialpose | geometry_msgs/msg/PoseWithCovarianceStamped | RViz or web gateway | dogzilla_localization_manager |
| /localization/cancel | std_msgs/msg/Bool | Web gateway | dogzilla_localization_manager |

Raw IMU orientation is explicitly unavailable. The controller's fused orientation convention is not trusted as a ROS world-frame orientation.

### Velocity and safety

| Topic | Type | Publisher or source | Consumer |
| --- | --- | --- | --- |
| /cmd_vel_nav_raw | geometry_msgs/msg/Twist | Nav2 controller/behavior server | Nav2 velocity smoother |
| /cmd_vel_nav_smoothed | geometry_msgs/msg/Twist | Nav2 velocity smoother | dogzilla_steering_guard |
| /cmd_vel_nav | geometry_msgs/msg/Twist | dogzilla_steering_guard | Twist Mux navigation channel |
| /cmd_vel_teleop | geometry_msgs/msg/Twist | Priority teleop or web zero-stop path | Twist Mux keyboard channel |
| /cmd_vel | geometry_msgs/msg/Twist | Twist Mux, direct teleop in mapping/drive, or web zero-stop path | dogzilla_safe_base |
| /safety/estop | std_msgs/msg/Bool | Web gateway | dogzilla_safe_base |

Twist Mux priorities are navigation 10 and keyboard 100, each with a 0.5 second input timeout. safe_base applies final speed clamps and its own 0.6 second watchdog after arbitration.

### Navigation support

| Topic | Type | Purpose |
| --- | --- | --- |
| /keepout_filter_mask | nav_msgs/msg/OccupancyGrid | Map-specific rasterized keepout polygons |
| /keepout_filter_info | nav2_msgs/msg/CostmapFilterInfo | Nav2 keepout-filter metadata |
| /navigation/diagnostics | std_msgs/msg/String | Warning-only health/stall observations as JSON |
| /navigation/tuning/status | std_msgs/msg/String | Read-only tuning-recorder state as JSON |
| /navigation/tuning/marker | std_msgs/msg/String | Operator marker added to the current tuning trial |

Diagnostics and tuning status are observational. Neither node publishes velocity, E-stop, Nav2 goals, cancellations or parameter updates.

### Vision

| Topic | Type | Publisher | Purpose |
| --- | --- | --- | --- |
| /camera/image_raw | sensor_msgs/msg/Image | usb_cam | Raw mono-camera stream |
| /camera/image_rect | sensor_msgs/msg/Image | image_proc in calibrated shadow mode | Rectified RTAB input |
| /camera/camera_info | sensor_msgs/msg/CameraInfo | Camera driver | Intrinsics and synchronized metadata |
| /vision/status | std_msgs/msg/String | dogzilla_vision | Mode, readiness, model coverage and disabled-action declaration |
| /vision/detections | std_msgs/msg/String | dogzilla_vision | JSON detections and non-executed proposals |
| /vision/danger_confirmed | std_msgs/msg/String | dogzilla_vision | Multi-frame confirmed person/hazard event |
| /vision/annotated/compressed | sensor_msgs/msg/CompressedImage | dogzilla_vision | Browser preview and alert image source |
| /vision/mode_command | std_msgs/msg/String | Web gateway | Validated detector configuration request |
| /vision/action_status | std_msgs/msg/String | Armed dogzilla_safe_base only | Execution state for explicitly armed vision control |

Normal vision and patrol perception always report action_output: disabled. Only the separate, interactively armed vision_control service can execute a fixed proposal through the serial manager.

### RTAB shadow namespace

| Topic | Purpose |
| --- | --- |
| /rtabmap_shadow/odom_input | Cartographer odom -> base_link converted to an isolated Odometry message |
| /rtabmap_shadow/info | RTAB processing, graph and loop-closure statistics |
| /rtabmap_shadow/map | Experimental RTAB map in its independent frame |

RTAB shadow uses /scan, rectified mono images and scan-matched odometry. It does not publish operational TF or control movement.

## Actions

| Action | Type | Client | Server |
| --- | --- | --- | --- |
| /navigate_to_pose | nav2_msgs/action/NavigateToPose | Web gateway | Nav2 BT navigator |
| /compute_path_to_pose | nav2_msgs/action/ComputePathToPose | Web gateway | Nav2 planner server |

The web gateway sends one NavigateToPose goal per task waypoint. A route preview calls ComputePathToPose and cannot move the robot.

## Services

| Service | Purpose |
| --- | --- |
| /start_trajectory | Start a live Cartographer localization trajectory |
| /finish_trajectory | Finish the current Cartographer trajectory |
| /get_trajectory_states | Discover frozen and active trajectories |
| /write_state | Persist a Cartographer PBStream during map saving |
| /dogzilla_safe_base/set_parameters | Change bounded speed, turn and drive-mode posture parameters |
| /dogzilla_safe_base/set_parameters_atomically | Apply autonomous speed/turn changes as one unit |
| /controller_server/set_parameters_atomically | Web autonomy profile update for Nav2 controller limits |
| /velocity_smoother/set_parameters_atomically | Web autonomy profile update for smoothed velocity limits |

Posture parameter changes are disabled in mapping/localization modes so the physical LiDAR scan plane remains consistent with base_link -> laser_frame.

## Nodes by launch

### hardware.launch.py

- MS200 scan driver and its working LiDAR frame

- /dogzilla_safe_base

- optional base_link -> imu_link static transform

### mapping.launch.py

- optional /dogzilla_imu_corrector

- /cartographer_node

- /cartographer_occupancy_grid_node

- optional RViz

### full_mapping.launch.py

Includes hardware.launch.py and mapping.launch.py in one process group.

### localization.launch.py

- optional /dogzilla_imu_corrector

- frozen-state /cartographer_node

- /dogzilla_localization_manager

- /dogzilla_tf_odometry

- /map_server

- /lifecycle_manager_localization

- optional RViz

### nav2.launch.py

- /controller_server

- /smoother_server

- /planner_server

- /behavior_server

- /bt_navigator

- /waypoint_follower

- /velocity_smoother

- /dogzilla_steering_guard

- /lifecycle_manager_navigation

### full_navigation.launch.py

Includes hardware, Twist Mux, localization and optionally Nav2. When Nav2 is enabled, it also starts /dogzilla_navigation_diagnostics and /dogzilla_navigation_tuning_recorder.

### vision.launch.py

- usb_cam through mono_camera.launch.py

- /dogzilla_vision

### vision_control.launch.py

Includes vision plus a startup-gated /dogzilla_safe_base that accepts no external velocity topic. It also enables the visualization-only robot description without LiDAR or IMU transforms.

### visual_shadow.launch.py

Includes the calibrated camera, raw vision processor, camera-only robot description and namespaced RTAB shadow launch. It starts no controller or LiDAR driver and must run beside the mapping container.

## Compose modes and devices

| Service/container | Main launch | Devices |
| --- | --- | --- |
| mapping / dogzilla_mapping | full_mapping.launch.py | /dev/ttyAMA0, /dev/ttyAMA1 |
| drive / dogzilla_drive | safe_base | /dev/ttyAMA0 |
| navigation / dogzilla_navigation | full_navigation.launch.py | /dev/ttyAMA0, /dev/ttyAMA1 |
| shadow / dogzilla_visual_shadow | visual_shadow.launch.py | /dev/video0 |
| vision / dogzilla_vision | vision.launch.py | /dev/video0 |
| vision_control / dogzilla_vision_control | vision_control.launch.py | /dev/ttyAMA0, /dev/video0 |
| perception / dogzilla_perception | vision.launch.py | /dev/video0 |
| web / dogzilla_web | web_gateway | None |

Serial-owning services are mutually exclusive. Mission Mode combines navigation, perception and web. Shadow mode combines mapping and shadow.

## Web API summary

The HTTP server listens on port 8080 by default. /healthz and static login assets are public; /api/v1/* requires X-Dogzilla-Password or the compatible Bearer form. Do not expose this port directly to the public internet.

Read endpoints provide state, map, tasks, named locations, patrol areas, keepout zones, hazards, alerts, alert photographs, the current annotated frame and server-sent events.

Write endpoints provide:

- delivery, generic route and patrol task creation;

- route/patrol preview;

- waypoint-mission (delivery) pause/continue and task cancellation;

- location, patrol-area and keepout persistence;

- initial-pose start and localization stop;

- map-switch preparation and completion;

- detector-mode changes;

- autonomous speed/turn changes;

- software E-stop and reset;

- tuning trial markers.

The authoritative implementation is dogzilla_slam/web_http.py. When adding an endpoint, keep validation and state transitions in the service/core layer, not in the browser JavaScript.

## QoS and freshness expectations

- /map and keepout masks use reliable, transient-local QoS so late subscribers receive the latest map.

- sensor data uses sensor-data QoS.

- vision status and diagnostic status use reliable, transient-local QoS.

- web task dispatch requires pose no older than 3 seconds.

- web battery start-gating requires a valid reading no older than 12 seconds.

- patrol requires vision status no older than 5 seconds.

Topic existence is not enough for readiness. Health and mission gates also check fresh content, transforms, action availability and validated state.

Source chapter: docs/OPERATIONS_RUNBOOK.md

# DOGZILLA S2 operations runbook

This is the operator procedure for the Raspberry Pi. Run these commands in a terminal on the Pi host, not inside a Docker container. Commands that can move the robot are marked MOVEMENT POSSIBLE.

The examples use the installed shell alias dogzilla. If the alias is not available, replace dogzilla with:

/home/pi/DOGZILLA/deploy/dogzilla-map

The absolute path runs the same host wrapper from any directory. Do not use docker exec for ordinary operation; the wrapper supplies the correct ROS domain, devices, mounts, logging, service handling and cleanup.

## Physical preflight

Before any mode that can move DOGZILLA:

- Put it on a clear, level, non-slip floor.

- Keep hands, cables and loose objects out of the legs.

- Keep the physical power button accessible.

- Confirm the battery is charged and not in firmware low-battery rest.

- Confirm no Yahboom mobile app, notebook or manual Python process is using /dev/ttyAMA0, /dev/ttyAMA1 or /dev/video0.

- Start with speed level 1 after any controller, navigation or pose change.

The browser emergency stop is useful but is not a certified physical E-stop. If motion is unsafe, use the physical power control.

## Setup and health checks

Open a terminal on the Pi and run:

cd /home/pi/DOGZILLA

dogzilla doctor

cd changes the terminal to the repository. dogzilla doctor checks Docker, Compose, the pinned base image, devices, disk space and desktop display. It does not start ROS or move the robot.

If dogzilla is not recognized, load the alias once:

source /home/pi/.bash_aliases

source evaluates the file in the current shell. The absolute filename avoids depending on the current directory. A new terminal normally loads the alias automatically.

Inspect the current deployment without starting a mode:

dogzilla status

status lists relevant Compose containers and, when one is active, important ROS topics. It does not command movement.

## Build and deployment model

After source changes and tests, build the local image:

dogzilla build

build builds dogzilla-mapping:humble from the pinned Dockerfile. It changes the local Docker image but does not start a robot mode, commit Git, push GitHub or upload the image. Source edits are not used by an existing container until this build completes and the affected mode is restarted.

There is no separate generic deploy command. Deployment means:

- build the image;

- stop the old mode;

- start the intended mode;

- verify status and logs.

Do not rebuild while the robot is walking or while a map save is in progress.

## Mapping a room

### Normal posture

MOVEMENT POSSIBLE after teleop starts. Start LiDAR-only mapping:

dogzilla start normal --headless

start launches the mapping service. normal leaves the controller at its normal firmware body height. --headless suppresses RViz, which is appropriate over SSH. On the Pi monitor, replace it with --rviz to open RViz. Omitting both flags automatically uses the detected display.

### Low posture

For a more stable, fixed scan plane:

dogzilla start low --headless

low performs a guarded change to 75 mm before mapping. Use the same posture when later localizing on this map. It is not a rest or servo-release command.

Add corrected IMU input only after a valid calibration and comparison trial:

dogzilla start normal --headless --imu

--imu enables the calibrated IMU Cartographer profile. It is optional; the current LiDAR-only profile is the operational baseline.

### Drive and observe

Open a second Pi/SSH terminal and run:

dogzilla teleop 1

teleop opens the keyboard controller. 1 is the initial linear speed level; number keys 1 through 9 change it while running. The - and + keys lower or raise turning level. Read the on-screen key guide. Release keys and stop immediately if the scan plane changes or motion becomes unstable.

Cover the room with slow loops and revisit distinctive areas. Avoid carrying the robot, changing body height, pushing it against walls or spinning rapidly.

On a monitor, view mapping in another terminal:

dogzilla rviz

rviz opens RViz against the active ROS domain. It requires a working local desktop display. It does not work in a display-less SSH session.

### Save and stop

Save the completed map while mapping is still active:

dogzilla save room1

save requests final Cartographer state and occupancy output. room1 is the map basename; use only letters, numbers, dot, underscore or dash. The command creates maps/room1.pbstream, maps/room1.pgm and maps/room1.yaml.

Then stop safely:

dogzilla stop

stop sends the layered controller stop, shuts down the active services and turns off the LiDAR through the patched driver path. It does not delete the saved map.

Verify the bundle:

ls -lh maps/room1.pbstream maps/room1.pgm maps/room1.yaml

ls lists the three files. -l shows details and -h uses readable sizes. All three are needed: PBStream for Cartographer localization and PGM/YAML for Nav2 and the web map.

## View an existing map without ROS

On the Pi desktop:

xdg-open /home/pi/DOGZILLA/maps/room1.pgm

xdg-open asks the desktop to open the occupancy image in its normal image viewer. The absolute path identifies the map. This shows the saved map but not live robot position.

To see the robot pose on the map, run localization, navigation or Mission Mode and use RViz or the web dashboard.

## Localization without autonomous navigation

Start a saved map for pose checking:

dogzilla localize room1 --headless

localize loads room1 without Nav2 goal execution. --headless disables RViz. The operator must set an approximate initial pose through the supported UI before localization is accepted. This mode should not autonomously walk.

Use global Cartographer matching instead of the operator pose gate only when the environment is distinctive enough:

dogzilla localize room1 --headless --match

--match asks Cartographer to search globally. It may be slower or ambiguous in repeated corridors; it is an explicit alternative, not proof of location.

Stop with dogzilla stop.

## Mission Mode

### Start

MOVEMENT POSSIBLE after a task is queued and localization is accepted.

dogzilla mission start room1 --headless

mission start coordinates navigation, camera perception and the web dashboard. room1 selects the PBStream/PGM/YAML bundle. --headless disables RViz. Startup checks ROS topics, localization components, Nav2, perception and web health and rolls the session back if a required component fails.

Equivalent short form:

dogzilla mission room1 --headless

The map name immediately after mission is treated as mission start room1.

For corrected IMU localization:

dogzilla mission start room1 --headless --imu

--imu selects the calibrated IMU profile. Do not use it merely because the sensor exists; compare it with the LiDAR-only baseline.

For global scan matching:

dogzilla mission start room1 --headless --match

--match bypasses the normal approximate-pose workflow and enables global matching. The robot must remain supervised until the map/scan result is credible.

### Open the dashboard

Get the current browser password and service status:

dogzilla mission password

dogzilla mission status

mission password prints the local dashboard password. mission status reports navigation, perception and web state. Neither command moves the robot. Use the Pi's address displayed by the startup/status output in a browser on the same trusted network.

### Set initial pose

In the map panel:

- select Initial pose;

- pan/zoom to the approximate robot location;

- click the approximate position and choose a heading, or custom heading;

- start pose matching;

- accept only when the corrected pose and scan overlay match the room;

- cancel matching and reposition the estimate if the correction/warning is unreasonable.

The initial click is a search centre, not an exact odometry value. The matcher can refine nearby translation and yaw, but it must not be trusted to turn a wrong room or repeated corridor into a correct pose.

### Queue missions

The web UI supports:

- a waypoint mission with one to ten ordered stops, each timed or manually continued (stored internally as task kind delivery for compatibility);

- polygon patrol with generated coverage points;

- map-specific keepout polygons;

- autonomous walking and turning speed sliders, rounded to levels 1–9.

For manual continuation, reaching a point changes the task to waiting. Press the large Continue button to send the next goal. A waiting task still owns the executor and blocks another queued task. Pause stops progress without discarding the task; Continue resumes it. Cancel ends it.

### Patrol-specific procedure

Before queuing a patrol:

- open Vision in the dashboard;

- select patrol and apply the mode;

- wait for object, person and face-detection readiness;

- confirm action output: disabled;

- create/select a patrol polygon and spacing on the active map;

- ensure no delivery/route is running, paused or waiting;

- queue the patrol.

Patrol detections notify the web and can save a photo; they do not currently stop the mission. Duplicate events are suppressed for the configured cooldown, and only the latest 25 alert records/photos are retained.

### Switch maps

When no task is moving the robot:

dogzilla mission switch-map room2

mission switch-map leaves web/camera services up and restarts the localization/navigation part with room2. A new initial pose and localization verification are required. Map-specific keepouts, locations and patrol areas are loaded for the selected map.

### Logs and stop

Follow the complete mission session:

dogzilla mission logs

mission logs follows navigation, perception and web logs together. Press Ctrl+C to stop following logs; it does not stop the mission.

Stop Mission Mode:

dogzilla mission stop

mission stop cancels task execution, stops web and perception, sends the navigation/controller stop path and shuts down navigation. It does not delete maps or task history.

## Low-level navigation development

Use only when the web task layer is not needed:

dogzilla navigate room1 --headless

navigate starts localization plus conservative Nav2. room1 selects the map and --headless disables RViz. It can execute Nav2 goals and therefore can move. Add --imu or --match only with the same meaning and cautions described above.

## Camera and vision

Test the camera without legs or LiDAR:

dogzilla camera-check 10

camera-check opens only /dev/video0 in an isolated test service. 10 is the sample duration in seconds. It validates image rate, timestamps and CameraInfo and then closes the camera.

Start non-actuating raw vision:

dogzilla vision raw

vision starts camera-only perception and the web view. raw shows the unaltered camera mode. This service has no controller serial device.

Start camera-only patrol detection:

dogzilla vision patrol

patrol selects danger, person and face detection. It reports observations but cannot move the robot.

Change a running vision/mission detector without reopening the camera:

dogzilla vision-mode patrol

vision-mode sends a mode request to the active perception node. patrol is the requested detector mode. Mode readiness should be checked in the web UI.

vision-control is a separate, armed hardware mode and can move the robot. It requires an interactive confirmation. Do not use it as a shortcut for Mission Mode or camera testing.

## IMU calibration and validation

Stop all DOGZILLA modes and place the robot so it can be held still in each requested pose. Then run:

dogzilla imu-calibrate

imu-calibrate temporarily owns the controller serial port and guides a six-pose calibration. It pauses the OLED service, refuses another serial owner, and atomically writes calibration/imu.json only after validation. Support the robot on a rigid fixture if hand shake prevents stable samples.

Validate for ten seconds:

dogzilla imu-check 10

imu-check starts the corrected IMU validation path. 10 is the duration in seconds. It checks rate, timestamp gaps/age and gravity magnitude. A mean near 9.6 m/s² is plausible for this sensor calibration and close to physical gravity; the pass/fail limits matter more than an exact 9.80665 reading.

## Stand, rest and LiDAR power

MOVEMENT POSSIBLE. With clear legs and valid battery telemetry:

dogzilla stand

stand loads all motors and invokes the controller's public animated stand-up action with a guard delay. It is not safe while the robot is constrained or held by the legs.

Do not use dogzilla rest as a power-button substitute. It is intentionally disabled because the public lie-down action and immediate torque release have not been proven identical to the private firmware power-button/low-battery sequence. rest-capture is observation-only and captured profiles are marked non-replayable.

Turn off a spinning LiDAR when no ROS mode should own it:

dogzilla lidar-off

lidar-off sends the MS200 motor-off path without starting the ROS stack. It does not turn off the leg servos. Do not run it while active mapping/navigation owns /dev/ttyAMA1; use dogzilla stop instead.

## Controller-only drive

MOVEMENT POSSIBLE. For slow controller tests without LiDAR:

dogzilla drive

dogzilla teleop 1

drive starts only the guarded controller bridge, so the LiDAR remains off. teleop 1 starts at the slowest level. Finish with dogzilla stop.

## Logs and diagnostics

Follow logs for the active non-mission mode:

dogzilla logs

logs follows the current service output. Ctrl+C exits the viewer without stopping the service.

Open a correctly configured diagnostic shell inside the active image:

dogzilla shell

shell opens Bash with ROS Humble, the built workspace and ROS_DOMAIN_ID=12 already configured. Prefer it to a raw docker exec. Exit with exit or Ctrl+D; exiting the shell does not stop the robot service.

Inside that shell, useful read-only checks include:

ros2 topic hz /scan

ros2 topic echo /battery_state --once

ros2 run tf2_ros tf2_echo map base_link

ros2 topic hz /scan measures LiDAR publication frequency until Ctrl+C. ros2 topic echo displays messages; /battery_state is the topic and --once exits after one message. ros2 run starts an installed executable; tf2_ros is its package, tf2_echo is the executable, and map base_link are the target and source frames whose transform is printed until Ctrl+C.

Check host device owners when startup reports a conflict:

sudo fuser -v /dev/ttyAMA0 /dev/ttyAMA1 /dev/video0

sudo requests host privileges. fuser lists processes using files; -v adds user/PID/access details. The three explicit device paths are the controller, LiDAR and camera. Inspect the PID before stopping anything; do not kill an unknown process blindly.

## Failure recovery

If startup fails:

- keep the robot stationary;

- read the exact terminal error;

- run dogzilla mission status for Mission Mode, otherwise dogzilla status;

- run the matching logs command;

- check device owners;

- use the matching safe stop command;

- correct the reported dependency/configuration problem;

- restart once and recheck readiness before queuing a goal.

Do not repeatedly restart around a serial conflict. Do not treat a Nav2 result status alone as the cause; inspect localization, TF/scan timing, controller logs and the task error from the same session.

If the terminal is lost while the robot mode remains active, reconnect to the Pi and run the status command, then the safe stop command. Docker services are not tied to the lifetime of an SSH terminal.

## Safe shutdown

Stop the active mode first:

dogzilla stop

For Mission Mode use dogzilla mission stop. Confirm the LiDAR is no longer spinning and the status output has no active robot service before turning off the Pi or robot.

The optional OS shutdown guard is installed once with:

dogzilla shutdown-install

shutdown-install installs the host shutdown integration that asks the robot stack to stop during OS shutdown. It changes host service configuration and is not a replacement for supervised shutdown.

## Backup checklist

Git backs up source and documentation, not Docker images or all runtime data. Before major changes, separately copy:

- maps/;

- calibration/;

- web/data/tasks.sqlite3 when task history/locations matter;

- required model weights and their provenance;

- selected logs/sessions/ reports;

- the output of docker image inspect dogzilla-mapping:humble if exact image identity matters.

Never put passwords, .env, private images or raw private datasets into the Git repository.

Source chapter: docs/COMPUTER_VISION.md

# DOGZILLA computer vision

This package turns Yahboom's Jupyter camera demonstrations into one reusable ROS 2 pipeline. The algorithms are based on the installed DOGZILLA samples and the lesson order on the official DOGZILLA study page.

## Implemented lessons

| Yahboom lesson | DOGZILLA mode | Current output |
| --- | --- | --- |
| Camera display | raw | Shared live camera and annotated JPEG |
| 8.1 Color recognition | color | Target centre, radius, area, and image offset |
| 8.2 Color tracking | color-track | Same continuous tracking measurements |
| 8.3 Color action | color-action | Allowlisted firmware-action proposal |
| 8.4 Face detection | face | Face boxes and normalized target offset |
| 8.5 Face tracking | face-track | Same continuous tracking measurements |
| 8.6 Watchdog | watchdog | Face-triggered handshake proposal from Yahboom's notebook |
| 8.7 QR recognition | qr | QR box and decoded text |
| 8.8 QR action | qr-action | Exact-phrase allowlisted action proposal |
| 8.11 Visual tracking | line | Lower-image line contour and steering offset |
| 8.11-8.12 Line following | line-follow | Steering/forward intent proposal |

The red, green, blue, and yellow HSV presets come from Yahboom lessons 8.1-8.3. The default line HSV range comes from the installed Samples/3_AI_Visual/11_12.followline/LineFollowHSV.text file.

The camera processor always publishes observations and read-only intent. It never opens /dev/ttyAMA0, sends velocity, runs an action group, or releases a servo. Every result contains action_output: disabled; every proposal contains executed: false and requires_explicit_arming: true. Arbitrary QR text is never interpreted as a command: only Yahboom's exact 19 labels can create a proposal.

## Architecture

/dev/video0

|

v

usb_cam (one owner) ---> /camera/image_raw ---> dogzilla_vision

|             |

v             v

/vision/detections   annotated JPEG

|             |

+------ web gateway

(password required)

Explicitly armed vision-control only:

/vision/detections ---> serial-manager policy ---> DOGZILLALib action

|                         /dev/ttyAMA0

+--> bounded line speed

Standalone Vision starts usb_cam and dogzilla_vision in one camera-only container. Visual-shadow mode already owns the camera for RTAB-Map, so it adds the processor to that same launch instead of starting another camera. Neither path gives the vision processor a serial device.

The separate vision_control service owns the camera and controller serial port in one guarded launch. Only the serial manager can execute a proposal. The launch is serial-free by default (armed:=false); the operator command is the only supported way to start it armed.

## Operator commands

Start a detection-only session and its authenticated dashboard:

dogzilla vision color red

vision selects the camera-only mode. color is the detector. red is the HSV preset. The command starts no legs, LiDAR, IMU, or motor serial process.

Start an explicitly armed Yahboom-style behavior:

dogzilla vision-control color-action red

dogzilla vision-control watchdog

dogzilla vision-control qr-action

dogzilla vision-control line-follow

The command explains the hazard and requires the exact typed confirmation ARM VISION before either device is opened. Armed control starts no LiDAR. It locks the slow profile, accepts no external velocity topic, requires five stable proposal frames, requires target release before repetition, enforces an eight-second action cooldown and guard, stops immediately when a line is lost, and retains the normal 0.6-second movement watchdog. Valid battery telemetry strictly above 25% is required.

Change a running standalone or RTAB visual-shadow detector:

dogzilla vision-mode face

dogzilla vision-mode color-track blue

dogzilla vision-mode color-action red

dogzilla vision-mode watchdog

dogzilla vision-mode qr

dogzilla vision-mode qr-action

dogzilla vision-mode line

dogzilla vision-mode line-follow

Open the dashboard URL printed by the command. Obtain its password with:

./deploy/dogzilla-web show-password

Stop the camera, disarm control, stop movement, and close the dashboard:

dogzilla stop

When Vision is the only active device, stop does not open the LiDAR serial port. Visual-shadow mode still follows its existing calibrated RTAB shutdown and database-save path.

## Safety boundary and remaining lessons

Lessons 8.3, 8.6, 8.8, and basic 8.11 line following now have a guarded path through the single /dev/ttyAMA0 manager. The camera and web processes still cannot execute anything directly. The dashboard shows a red ARMED state when that separate session is active.

Lesson 8.12 obstacle crossing is not armed: it changes gait and continues forward after detecting an obstacle, so it requires its own physical clearance and stop tests. Lessons 8.9 climb, 8.10 kick/play-ball, 8.13 action learning, and 8.14 synchronized teaching also remain outside automated control. The original samples open DOGZILLALib directly, bypassing the ROS battery lockout, limits, and watchdog; that architecture is intentionally not copied.

Source and synthetic validation do not replace physical acceptance. Test one firmware action at a time on a supported, clear floor before treating armed vision control as production-ready.

Source chapter: docs/URDF_RTABMAP_MONO.md

# URDF and monocular RTAB-Map framework

## Current status

This is a deployed but calibration-gated shadow workflow. Exact Humble versions of the camera, image processing, Xacro, robot-state-publisher, and RTAB-Map dependencies are installed in dogzilla-mapping:humble. Normal mapping and navigation launch files remain unchanged. Only dogzilla shadow can start the separate visual service, and it refuses before hardware startup when either required camera calibration file is absent or invalid.

The framework deliberately separates two jobs:

calibrated mono image + CameraInfo -------------------+

MS200 /scan ------------------------------------------+--> RTAB-Map

base_link -> camera_optical_frame TF -----------------+       |

Cartographer odom -> base_link TF                     |       +--> database

-> namespaced odometry adapter                      |       +--> maps

-> /rtabmap_shadow/odom_input ----------------------+       +--> no TF

Cartographer remains the owner of the operational map -> odom transform. RTAB-Map uses the monocular camera for visual features and place recognition, then uses scan-matched odometry and the MS200 scan to retain metric scale. During mapping, the isolated adapter converts Cartographer's existing odom -> base_link TF into /rtabmap_shadow/odom_input; it does not publish or modify TF. A single monocular camera cannot provide dependable absolute scale by itself.

The two map views are intentionally separate. dogzilla rviz shows Cartographer's /map with the URDF. While visual shadow mode is running, dogzilla shadow-rviz shows RTAB's /rtabmap_shadow/map in its own rtabmap_shadow_map frame. The deployment does not fake an identity transform between two independently optimized map frames.

This is an appearance-enhanced 2D LiDAR map, not dense 3D reconstruction. Reliable dense geometry would require depth, stereo, or another 3D sensor; monocular RGB is mainly valuable here for recognizing previously seen places.

## Files

- urdf/dogzilla_s2.urdf.xacro defines the body and the existing base_link, laser_frame, and imu_link frames. It adds camera_link and a REP-103 camera_optical_frame.

- urdf/dogzilla_leg.xacro defines 12 revolute joints with the exact names published by safe_base.py.

- launch/robot_description.launch.py is a standalone robot-state-publisher launch with enabled:=false as its default.

- config/rtabmap_mono_shadow.yaml configures RGB-only visual input together with /scan and scan-matched odometry. It sets publish_tf: false.

- launch/rtabmap_mono_shadow.launch.py starts no process unless its explicit safety gate is enabled. Its output stays under /rtabmap_shadow and its database defaults to /logs/rtabmap_mono_shadow.db. Its read-only odometry adapter can be disabled when an existing odometry message is selected.

- config/mono_camera.yaml is the tested 640x480, 30 Hz MJPEG camera profile.

- deploy/Dockerfile checksum-pins the upstream usb_cam frame-draining fix; the stock Humble 0.8.0 timer otherwise returned nearly one-second-old frames from this camera's 120 Hz V4L2 queue.

- launch/mono_camera.launch.py owns /dev/video0 and optionally rectifies.

- launch/camera_calibration.launch.py combines the tested camera profile with the official ROS intrinsic-calibration GUI and shuts the camera down when the GUI exits.

- launch/visual_shadow.launch.py combines the gated camera, camera TF and RTAB launch while omitting duplicate LiDAR and IMU transforms.

- dogzilla_slam/camera_model.py rejects missing, placeholder, wrong-size, or unmeasured calibration before deployment starts.

- dogzilla_slam/camera_validate.py validates live frames, timing and calibrated CameraInfo.

- dogzilla_slam/shadow_validate.py proves that image, LiDAR, and scan-matched odometry timestamps overlap and that RTAB is publishing processed-node statistics. Topic existence alone is not treated as success.

## Camera contract

The Yahboom application and ROS camera cannot own /dev/video0 together. The operator command stops the Yahboom application through its normal Ctrl-C path, refuses unknown competing owners, and then makes usb_cam the sole ROS owner:

| Topic | Type | Requirement |
| --- | --- | --- |
| /camera/image_rect | sensor_msgs/msg/Image | Rectified monocular image |
| /camera/camera_info | sensor_msgs/msg/CameraInfo | Real intrinsic calibration, matching timestamps |

The image frame_id should be camera_optical_frame. The camera calibration must be produced at the exact deployed resolution and must not contain the all-zero placeholder matrices commonly emitted by an uncalibrated driver.

dogzilla camera-check 12 verifies raw data without calibration. It must not be used as evidence that rectification or RTAB-Map is ready. The shadow command additionally requires non-zero intrinsic matrices and a measured mount transform before it opens the camera.

dogzilla camera-calibrate BOARD_SIZE SQUARE_METRES writes the GUI COMMIT to a temporary path, validates it, then atomically installs camera.yaml; a cancel, crash, malformed result, or wrong resolution cannot replace a prior file. dogzilla camera-extrinsics --measured X Y Z ROLL PITCH YAW generates the mount YAML from metres and degrees, avoiding manual YAML formatting errors.

## Measurements still required

The current sensor transforms that already work for 2D mapping are preserved:

- base_link -> laser_frame: (0.000, 0.000, 0.180) metres

- base_link -> imu_link: (0.085, 0.000, 0.070) metres

Everything else in the mechanical model is provisional. Before operational use, measure and verify:

- The camera translation and roll/pitch/yaw relative to base_link.

- Chassis dimensions, leg link lengths, and joint origins.

- Which physical corner Yahboom calls legs 1 through 4.

- Every motor's axis direction and zero offset against /joint_states.

- Joint limits without commanding the robot from the URDF.

The model is currently suitable only as a TF and visualization framework. It must not be treated as a control, collision, or kinematics model.

## Remaining activation gates

- Measure the real checkerboard and run dogzilla camera-calibrate to generate calibration/camera.yaml for dogzilla_mono at exactly 640x480.

- Measure the camera translation and RPY relative to base_link, then run dogzilla camera-extrinsics --measured ... with the physical values.

- Run dogzilla shadow --headless, then dogzilla shadow-check 10 while the robot is stationary.

- In a second test, run dogzilla shadow-route-check 120 1.0 in one terminal and dogzilla teleop 1 in another. Drive a closed route, revisit a textured starting view, and stop near the start. The read-only observer must report at least one global RTAB loop closure before relying on the visual database.

- Run dogzilla stop, then dogzilla shadow-db-check. The saved database must pass SQLite integrity, RTAB 0.23.7 schema, node-data, statistics, and link consistency checks.

Existing PGM/YAML/PBStream maps remain usable because their frame names and working sensor transforms have not changed. They contain no historical camera frames, so evaluating visual loop closure will require a new camera-enabled recording or mapping run after deployment.

## Validation status

Current validation passes:

- 115 source and deployment tests plus ROS Python/launch style validation

- installed-image package discovery for the pinned camera overlay and DOGZILLA package

- Xacro expansion to 21 links/20 joints normally and 19/18 in shadow mode

- live base_link -> camera_optical_frame TF with LiDAR/IMU duplicates omitted

- physical raw camera data at about 28 Hz, 640x480, correct frame ID, monotonic timestamps, about 0.016-second mean age, and low delay jitter

- RTAB 0.23.7 launch-time parameter acceptance, intended subscriptions, isolated odometry, no devices, no privilege, no /cmd_vel, publish_tf false, no observed TF messages, SQLite database creation, and clean SIGINT save/exit

- missing-calibration refusal before any mapping or hardware service starts

- guarded intrinsic/extrinsic writers that cannot replace an existing file with a cancelled, malformed, implausible, or unacknowledged measurement

- read-only route acceptance logic that distinguishes global loop closures from proximity detections and rejects insufficient travel, poor return, and large odometry jumps

- atomic JSON health and route reports under the current session log, including failed measurements and rejection reasons

- clean Docker SIGINT persistence and read-only integrity checking across an RTAB database restart

Still unverified are calibrated rectification quality, combined physical camera/scan/odometry synchronization, CPU/temperature during the full stack, visual features while moving, real loop-closure precision, and long-duration database behavior. Those require the two physical camera calibration files; the deployment does not invent them or bypass the gate.

Source chapter: docs/DEVELOPMENT_ROADMAP.md

# DOGZILLA S2 development roadmap

This roadmap orders future work by safety and dependency, not by novelty. The robot already maps, localizes, navigates, runs web tasks and reports camera observations. The next developer should improve repeatability and evidence before adding more autonomous behavior.

Status terms used here:

- operational: deployed and exercised on this robot;

- implemented: source/tests exist, but more physical acceptance is needed;

- prepared: reversible source candidate exists and is inactive;

- experimental: isolated from operational control;

- blocked: required evidence or hardware is missing.

## Safety rules for every milestone

Every change that can affect motion follows the same gates:

- write a measurable acceptance criterion before changing parameters;

- preserve a known-good source/image and robot-specific data backup;

- run pure tests and configuration contracts;

- build once from clean pinned inputs;

- check stationary topics, TF, battery and command outputs;

- trial at speed level 1 on a clear floor with physical power accessible;

- change one variable group at a time;

- save the complete session and compare it with the baseline;

- promote, revise or revert based on evidence;

- document the deployed image ID and configuration.

Never weaken the command watchdog, low-battery protection, serial ownership, goal/map validation or physical supervision to make an acceptance test pass.

## P0: establish a dependable baseline

### P0.1 Navigation stability trial

Status: prepared, inactive.

Problem: observed missions showed Cartographer work-queue growth, reduced scan processing and unnecessary replanning. Steering oscillation must not be tuned blindly while localization timing is degraded.

Prepared source lives in plans/navigation-stability/. It proposes:

- 0.5 Hz instead of 1 Hz global replanning;

- fewer Cartographer background threads and constraints;

- less frequent pose-graph optimization and state/submap publication;

- two retained localization submaps;

- 0.50 second Nav2 transform tolerance.

It intentionally leaves scan rate, walking speed, obstacle layers, footprint, collision handling and emergency stop unchanged.

Acceptance evidence:

- /scan remains near the MS200's expected 10 Hz without growing age;

- Cartographer work queue stays bounded over a representative route;

- map -> odom -> base_link remains continuous;

- controller deadline warnings and Nav2 transform failures do not increase;

- cross-track error and left/right reversal count improve over the baseline;

- obstacle marking and stopping remain intact;

- apply and revert produce reproducible source states.

Do not combine the first trial with IMU, speed, footprint or camera changes.

### P0.2 Navigation recorder and result classification

Status: implemented; expand evidence library.

The recorder should continue to capture only tuning evidence:

- planned path and replans;

- raw, smoothed, guarded and final velocity commands;

- scan/TF/odometry timing;

- map pose, odom pose and cross-track error;

- front/side LiDAR clearances;

- task/Nav2 action result and cancellation source;

- active Nav2/Cartographer/speed parameters.

Next work is not “record more topics.” Add a small report that labels each trial as pose loss, planner failure, controller oscillation, obstacle stop, stall inference, operator cancel or successful arrival. Store the test layout, initial pose and image/config revision with every report.

Acceptance evidence:

- the same log produces the same summary;

- report timestamps align with ROS time;

- no recorder output can publish velocity, cancel a goal or alter parameters;

- reports remain bounded and cannot fill the Pi filesystem.

### P0.3 Physical collision/stall protection

Status: software warning implemented; collision certainty blocked by hardware.

The robot has no wheel encoders, bumper or leg contact sensor. Comparing a command with scan-matched motion can warn that it is stalled, but it cannot prove contact and can be wrong during scan loss, carpet slip or body sway.

Recommended design:

commanded motion + scan odometry + LiDAR clearance + localization health

|

v

warning / confidence state

|

operator-visible alert and bounded stop policy

For a dependable contact stop, add and independently power-test a bumper, foot/contact switch, motor-current/servo-load signal or other physical sensor. Do not create a sensitive “cancel on small odometry difference” rule: false positives can leave the robot stranded and false negatives can permit pushing.

Acceptance evidence:

- no warning while standing, turning normally or walking over representative floor transitions;

- repeatable warning for a safely restrained forward-motion test;

- sensor/timing failure is distinguishable from contact;

- stopping is latched, visible and requires deliberate recovery;

- the policy never bypasses safe_base.

### P0.4 Automatic patrol perception handshake

Status: manual workflow operational; automatic handshake not implemented.

Currently a patrol remains queued until the operator sets Vision mode to patrol. Implement a gateway state machine, not a browser-only convenience:

- a patrol is selected;

- request patrol through /vision/mode_command;

- wait for matching /vision/status with a bounded timeout;

- verify full detector coverage, confirmation thresholds and action_output: disabled;

- dispatch only while that exact readiness remains healthy;

- fail visibly if perception changes or becomes stale.

Delivery and ordinary route tasks may default to raw vision, but must not silently arm a camera action mode. Confirmed dangers continue to notify and save a photo; they do not stop a mission unless a separately reviewed policy is added.

Acceptance evidence:

- patrol starts from raw mode without manual detector selection;

- timeout/missing model leaves the task queued or failed and stationary;

- stale status cannot satisfy readiness;

- changing out of patrol during execution causes an explicit safe task state;

- no camera event directly commands motion.

### P0.5 Backup and restore drill

Status: documented, not yet proven as a full restore.

Git is only the source backup. Create a dated, checksummed robot backup for:

- map bundles;

- calibration files;

- mission database and selected alert images;

- model weights plus source/version/license metadata;

- deployed Git revision, dirty diff and Docker image ID;

- selected acceptance logs.

Test restore into a temporary directory and validate SQLite and map bundles. Do not include passwords, environment secrets or private raw datasets in Git.

Acceptance evidence:

- a second operator can restore a map/calibration/database without guessing;

- checksums detect truncation;

- PBStream/PGM/YAML basenames remain consistent;

- restore cannot overwrite live data without an explicit backup and choice.

## P1: calibrate the physical model

### P1.1 Camera intrinsics

Status: temporary calibration exists; real checkerboard calibration still required.

OpenCV is the image-processing library and calibration implementation. A camera model is the measured result OpenCV needs to remove lens distortion and project image rays. They work together; installing OpenCV cannot replace measuring this DCX-BAT53G2V2 camera/lens at the deployed 640 x 480 mode.

Use a rigid checkerboard with measured square size, fill the image area and angles, commit only a low-error calibration and repeat after focus, lens or resolution changes.

Acceptance evidence:

- calibration is for dogzilla_mono, 640 x 480;

- reprojection error is recorded and rectified straight lines look straight;

- CameraInfo timestamps/frame IDs match the images;

- the result replaces camera.temporary.yaml only after review.

### P1.2 Camera extrinsics

Status: framework present; physical values must be measured/verified.

Measure base_link -> camera_link XYZ and RPY from the base-link definition in calibration/BASE_LINK.md. Do not optimize guessed extrinsics until they “look right”: that can absorb time offset or intrinsic errors into a false transform.

Acceptance evidence:

- mounting reference and uncertainty are written down;

- TF has one publisher and no cycle;

- projected stable features do not shift incorrectly during controlled turns;

- visual shadow remains non-controlling.

### P1.3 URDF physical measurements

Status: experimental visualization framework.

The current URDF provides body, LiDAR, IMU and camera frame structure. Before using legs for collision, kinematics or control, measure:

- body/link dimensions and masses;

- hip positions and axis directions;

- upper/lower leg lengths;

- servo zero offsets, signs and joint limits;

- motor-ID-to-corner/joint mapping;

- center of mass and contact geometry.

Acceptance evidence:

- a measurement table includes tool, method and uncertainty;

- live joint states move the correct rendered joint in the correct direction;

- all four legs match physical poses through their safe range;

- a reviewer signs off before URDF is used for control or collision stopping.

### P1.4 IMU comparison

Status: calibration and correction operational; fusion optional.

The calibrated sensor passed rate/gravity/timestamp checks, but IMU-enabled navigation has not consistently outperformed LiDAR-only localization. Keep the two profiles. Record matched routes under the same map, posture, speed and initial pose.

Acceptance evidence:

- six-pose calibration passes without hand-motion contamination;

- stationary gyro bias and gravity direction remain within limits;

- no timestamp jumps or stale samples;

- heading/cross-track error improves over LiDAR-only in repeated trials;

- if it does not improve, LiDAR-only remains the default.

### P1.5 Initial-pose validation dataset

Status: wide nearby search implemented; thresholds need multi-room evidence.

Record correct/incorrect candidate poses in:

- open rooms;

- repeated corridors;

- furniture changes;

- partial occlusion;

- several yaw errors and position offsets up to the supported radius.

Evaluate endpoint match, mapped-ray contradiction, coverage, correction size and repeat stability. A 30% alignment threshold alone is not proof of location. The matcher should refine an approximate area and warn on excessive correction, not teleport between ambiguous rooms.

Acceptance evidence:

- documented false-accept and false-reject rates;

- furniture does not force unreasonable rejection;

- a clearly wrong location is rejected in every selected test;

- the UI shows searched area, best correction and confidence limitations;

- cancellation immediately stops matching without stopping Mission Mode.

## P2: mature autonomous missions

### P2.1 Task state and manual continuation

Status: implemented; physical workflow acceptance needed.

Exercise deliveries and up-to-ten-point routes with timed waits and manual continuation. Validate pause, continue, cancel, web reconnect and process restart at every step. A waiting task must remain the only active task; restart must never silently resume motion.

Acceptance evidence:

- every transition is persisted and visible;

- repeated Continue clicks cannot skip a point;

- cancel sends zero movement and ends the action;

- browser disconnect does not create duplicate goals;

- invalid/stale battery telemetry is visible but does not masquerade as 0%; a confirmed low valid battery still prevents unsafe movement.

### P2.2 Dynamic obstacle behavior

Status: live LiDAR costmaps operational; recovery deliberately limited.

Test people, chairs and movable furniture at LiDAR height. The local costmap is temporary and rebuilt each mission; static map and per-map keepouts persist. Tune observation persistence, inflation and progress checking only with log evidence. Keep automatic spin and backup disabled until separately accepted.

Objects above or below the 2-D scan plane are not dependable LiDAR obstacles. The monocular camera can classify an item but cannot provide safety-grade depth from a single frame.

### P2.3 Map switching

Status: implemented; transactional acceptance needed.

mission switch-map preserves web and camera while replacing localization and navigation. Validate rollback when a map bundle is incomplete, require a new initial pose, clear old pose/costmaps, and load only the selected map's locations, patrols and keepouts.

Acceptance evidence:

- no old-map goal can dispatch after switch;

- failed switch returns to a clearly named safe state;

- current map/revision is visible in API and UI;

- active/waiting tasks block or deliberately resolve before switching.

### P2.4 Alert semantics and retention

Status: implemented baseline.

Keep duplicate suppression based on label, spatial overlap and time window; retain at most 25 alert photos. Add operator acknowledgement/export only if it has a real use case. Face detection must remain presence detection, not identity recognition, unless privacy, consent and retention are redesigned.

Pretrained open-vocabulary labels are best-effort observations. Small screws, shards, wires and transparent glass are difficult at distance and must never be advertised as guaranteed floor safety.

## P3: experiments isolated from control

### P3.1 Monocular RTAB-Map shadow

Status: experimental and calibration-gated.

RTAB shadow may record visual features, database statistics and loop closures beside Cartographer. It must keep:

- no controller or LiDAR serial device;

- no /cmd_vel publication;

- no operational TF authority;

- its own namespace/database;

- explicit camera intrinsics/extrinsics gates.

Do not replace the LiDAR localization stack unless repeated route tests show a clear benefit and all control consumers are migrated through a reviewed design. Monocular imagery does not magically produce metric depth; scale presently comes from external motion/scan information.

### P3.2 Semantic map annotations

Status: future.

If furniture or engineering objects need to appear on the map, store semantic annotations separately from the immutable occupancy map. Each annotation should include map name/revision, class, pose, uncertainty, source, timestamp and expiry. Movable furniture belongs in live costmaps or expiring annotations, not permanently painted into PBStream/PGM as a wall.

## Firmware work boundary

The microcontroller firmware source is not present. The repository controls it through the installed DOGZILLALib serial protocol. Future public-protocol work must extend the single safe_base owner and document register/scaling evidence.

Blocked until vendor/source/analyzer evidence exists:

- firmware-identical power-button or low-battery rest trajectory;

- safe timed torque release after that trajectory;

- internal servo loops, current limits and thermal behavior;

- private battery thresholds/state machine;

- flash/upgrade implementation.

Do not replay captured serial packets to discover these behaviors while the robot is loaded. A profile capture is evidence, not execution approval.

## Work explicitly not recommended

- a second /dev/ttyAMA0 reader for IMU, battery or joints;

- immediate unload_allmotor() after public action 1;

- enabling movement from ordinary camera detections;

- treating monocular class boxes as measured obstacle distance;

- tuning speed to hide localization/controller instability;

- painting movable furniture into the static occupancy map;

- enabling URDF leg control from provisional geometry;

- automatic spin/backup recovery without physical trials;

- weakening localization acceptance because one furnished room fails;

- committing Docker layers, credentials, raw private datasets or generated runtime databases to Git.

## Suggested next three development cycles

### Cycle 1: measurement

- Run the inactive navigation-stability candidate as an A/B trial.

- Complete the real camera checkerboard and mount calibration.

- Build a multi-room initial-pose validation dataset.

- Perform and document a backup/restore drill.

### Cycle 2: reliability

- Promote or revert the navigation candidate from evidence.

- Implement the automatic patrol perception handshake.

- Complete task pause/wait/reconnect physical acceptance.

- Design a real physical contact/stall sensor interface.

### Cycle 3: capability

- Validate transactional map switching.

- Add semantic annotations without modifying the occupancy authority.

- Evaluate calibrated RTAB shadow route/loop-closure reports.

- Decide whether the IMU profile earns promotion or remains optional.

Each cycle should end with updated documentation, test results, deployed image identity, physical acceptance notes and an explicit rollback path.